RS-232C standard command specification

MODEL:

Revision  1.02

Copyright(C)2007 YAMAHA CORPORATION All Rights Reserved

RX-V1800 RX-V3800 HTR-6190

| Rev. | Date | Contents |
| --- | --- | --- |
| 1.01 | 10/29/2007 |  |
| 1.02 | 02/03/2008 | The command specification of the input name of 5.4 is changed. |

RX-Vx800 RS-232C Protocol

1. Outline

RX-Vx800 in this protocol refers to the RX-V1800 and RX-V3800.

1.1 Connection

5 wire system

RX-Vx800

Slave

TxD(PIN3)

RxD(PIN2)

GND(PIN5)

CTS(PIN8)

RTS(PIN7)

Transpose	--------->

Receive	<---------

Ground	----------

Permit to send data	<---------

Request to send data	--------->

RxD(PIN2)

TxD(PIN3)

GND(PIN5)

RTS(PIN7)

CTS(PIN8)

HOST.

master

1.2

*When not connected, data transmission to RX-Vx800 is prohibited (CTS port pull down).

RS-232C Settings

*Full duplex, start-stop synchronization communication

Baud rate

Data bits

Parity

Stop bit

Handshaking

9600bps

8

No

1bit

Hardware

*The RX-Vx800 RTS port outputs at low level when the AC plug is disconnected.

If RTS output stays low even when the AC plug is connected, a problem may occur.

1.3 Data block timeout

It takes the RX-Vx800 a maximum of 500msec to send one data block. If a complete data block is not received within 500msec,

cancel the transaction. A problem may occur.

3

2. Start Commands

2.1 Starting Communication

READY

RX-Vx800 RS-232C Protocol

The Ready command is the first command to be sent to the RX-Vx800 at the start of communication. TOUT0 - 2 in the Ready Command sets communication timeout.

RX-Vx800

CONFIG

Control Host

RX-Vx800 sends a Configuration command (Model ID, software version and setting data) to the host in reply to the Ready command.

The RX-Vx800 will send a Configuration command within 1 sec. after receiving a Ready command from the host. If not, send a Ready Command again (max 5 times).

If the RX-Vx800 won't send a Configuration command after the fifth retry, cancel the transaction because there may be a problem.

READY

CONFIG

DC1

DC2

TOUT0

TYP0

TOUT1

TYP1

TOUT2

TYP2

ETX

TYP3

TYP4

VER

L0

L1

DT0 -DT255

SUM0

SUM1

ETX

*TYPx

*VER 
*SUM

Model ID = "R0226" (RX-V1800)  Model ID = "R0227" (HTR-6190)  Model ID = "R0225" (RX-V3800) Software Version

The sum of all data except for the header and footer

| function name | function | data (ASCII) | range (HEX) |
| --- | --- | --- | --- |
| TOUT0 - 2 | communication timeout | 0 - 9, A - F | 0 - 0xFFF |

*timeout between the header and the footer *timeout=0 means no timeout

| function name | function | data (ASCII) |  | range (HEX) |
| --- | --- | --- | --- | --- |
| TYP0 - 4 | model ID | 0 - 9, A - F |  | voluntary |
| VER | software version | A - Z |  | voluntary |
| L0 - 1 | data length | 0 - 9, A - F |  | 0 - 0xFF |
| DT0 - 255 | data | 0 - 9, A - F |  | 0 - 0xF |
| SUM0 | upper 4 bit of SUM | 0 - 9, A - F |  | 0 - 0xF |
| SUM1 | lower 4 bit of SUM | 0 - 9, A - F |  | 0 - 0xF |

4

RX-Vx800 RS-232C Protocol

*Data Structure of Configuration Command

When the power is OFF, only DT0,1,…,9 are sent to the Host.

data

Report

| DT0 | - | Fixed Baud Rate Don't care ( '@' ) Fixed Receive Buffer Don't care ( 'E' ) Fixed Receive Buffer Don't care ( '0' ) Fixed Command Timeout Don't care ( '1' ) Fixed Command Timeout Don't care ( '9' ) Fixed Command Timeout Don't care ( '0' ) Fixed Handshaking Don't care ( '0' ) |  |
| --- | --- | --- | --- |
| DT1 | - |  |  |
| DT2 | - |  |  |
| DT3 | - |  |  |
| DT4 | - |  |  |
| DT5 | - |  |  |
| DT6 | - |  |  |
| DT7 | 00 | 0 / 2 0 - 7 0 - F 0 - F 0 - 8 0 - 1 0 - E 0 / 1 0 - F 0 - F 0 - F 0 - F 0 - F | System 0: OK / 1: Busy / 2: Standby Power Main Room / Zone2 / Zone3 Power Status (All off , All on, Main on…) Input Upper 4 bit Input Lower 4 bit Audio Select 0: Auto / 3: COAX/OPT / 4: Analog / 8:HDMI Audio Mute 0: Off / 1: On Zone2 Input 0: PHONO / 1: CD / 2: TUNER / 3: CD-R / 4: MD/TAPE / 5: DVD / 6: DTV/CBL 9: VCR / A: DVR / C: V-AUX / D: NET/USB* / E:XM** / F: BD/HD DVD *Only RX-V3800 **Only USA & Canada Zone2 Mute 0: Off / 1: On Master Volume Upper 4 bit Master Volume Lower 4 bit Zone2 Volume Upper 4 bit Zone2 Volume Lower 4 bit Program Upper 4 bit |
| DT8 | 20 |  |  |
| DT9 | 21 |  |  |
| DT10 |  |  |  |
| DT11 | 22 |  |  |
| DT12 | 23 |  |  |
| DT13 | 24 |  |  |
| DT14 | 25 |  |  |
| DT15 | 26 |  |  |
| DT16 |  |  |  |
| DT17 | 27 |  |  |
| DT18 |  |  |  |
| DT19 | 28 |  |  |
| DT20 |  | 0 - F Program Lower 4 bit 0 / 1 Effect 0: Off / 1: On |  |
| DT21 | 28 |  |  |
| DT22 | 2D | 0 - 6 1 - 2 0 - 4 0 - 4 0 - 7 | Extended Surround 0: Off / 1: EX/ES / 3: Auto / 4: EX / 5: PLIIx Movie / 6: PLIIx Music 7: Neural THX Short Message 1: On / 2: Off Sleep 0: 120 / 1: 90 / 2: 60 / 4: 30 / 4: OFF Tuner Preset Page 0: Page A / 1: Page B / 2: Page C / 3: Page D / 4: Page E Tuner Preset Number 0: No.1 / 1: No.2 / 2: No.3 / 3: No.4 / 4: No.5 / 5: No.6 / 6: No.7 / 7: No.8 |
| DT23 | 2B |  |  |
| DT24 | 2C |  |  |
| DT25 | 29 |  |  |
| DT26 | 2A |  |  |
| DT27 | 8B | 0 - 2 | Night Mode Upper 4 bit 0: OFF / 1:Cinema / 2:Music Deletion |
| DT28 | 8B | 0 - 2 Night Mode Parameter Lower 4 bit 0:Low / 1:Mid / 2:High Deletion |  |
| DT29 | 2E | 0 / 1 0 / 1 0 - F 0 - F 0 - F 0 - F 0 - F 0 / 1 0 / 1 0 - F 0 / 1 0 - 2 0 - 2 0 - 3 0 - 3 | Speaker Relay A 0: Off / 1: On Speaker Relay B 0: Off / 1: On Format Upper 4 bit Format Lower 4 bit Sampling Upper 4 bit Sampling Lower 4 bit Channel Front/Rear 0: 1+1 / 1: 1/0 / 2: 2/0 / 3: 3/0 / 4: 2/1 / 5: 3/1 / 6: 2/2 / 7: 3/2 / 8: 2/3 / 9: 3/3 A: 2/4 / B: 3/4 / C: MLT / F: --- Head Phone 0: Off / 1: On Tuner Band 0: FM / 1: AM Channel LFE 0: 0.1 / 1: 0.2 / F: --- Trigger1 Output 0: Low / 1: High Decoder Mode 0: Auto / 1:DTS / 2:AAC* *Only Japanese Model Dual Mono 0: Main / 1:Sub / 2:All (Only Japanese Model) Trigger1 Control 0: All (Zone1-3) / 1:Main / 2: Zone2 / 3: Zone3 Don't care Trigger2 Control 0: All (Zone1-3) / 1:Main / 2: Zone2 / 3: Zone3 |
| DT30 | 2F |  |  |
| DT31 | 10 |  |  |
| DT32 |  |  |  |
| DT33 | 11 |  |  |
| DT34 |  |  |  |
| DT35 | 12 |  |  |
| DT36 | 34 |  |  |
| DT37 | 35 |  |  |
| DT38 | 13 |  |  |
| DT39 | 36 |  |  |
| DT40 | 5E |  |  |
| DT41 | 39 |  |  |
| DT42 | 3A |  |  |
| DT43 |  |  |  |
| DT44 | 3B |  |  |
| DT45 | 3C | 0 / 1 Trigger2 Output 0: Low / 1: High 0 / 1 Speaker B set 0: Main / 1: Zone B |  |
| DT46 | 3D |  |  |
| DT47 | 3E | 0 - 3 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F | Zone 2 Amplifier 0: EXT / 1: INT: Speaker[1] / 2: INT: Speaker[2] / 3: INT Both Level Front R Upper 4 bit Level Front R Lower 4 bit Level Front L Upper 4 bit Level Front L Lower 4 bit Level Center Upper 4 bit Level Center Lower 4 bit Level Surround R Upper 4 bit Level Surround R Lower 4 bit Level Surround L Upper 4 bit Level Surround L Lower 4 bit Level Surround Back R Upper 4 bit Level Surround Back R Lower 4 bit Level Surround Back L Upper 4 bit |
| DT48 | 40 |  |  |
| DT49 |  |  |  |
| DT50 | 41 |  |  |
| DT51 |  |  |  |
| DT52 | 42 |  |  |
| DT53 |  |  |  |
| DT54 | 43 |  |  |
| DT55 |  |  |  |
| DT56 | 44 |  |  |
| DT57 |  |  |  |
| DT58 | 45 |  |  |
| DT59 |  |  |  |
| DT60 | 46 |  |  |
| DT61 |  | 0 - F Level Surround Back L Lower 4 bit |  |

5

RX-Vx800 RS-232C Protocol

| DT62 | 47 | 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - 4 0 - 7 0 - 2 1 - 3 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - F 0 - A 0 / 1 0 / 1 0 - 4 | Level Presence R Upper 4 bit Level Presence R Lower 4 bit Level Presence L Upper 4 bit Level Presence L Lower 4 bit Level Subwoofer Upper 4 bit Level Subwoofer Lower 4 bit XM Preset Page 0: Page A / 1: Page B / 2: Page C / 3: Page D / 4: Page E (Only USA & Canada) XM Preset Number (Only USA & Canada) XM Search Mode 0: All CH / 1: Category / 2: Preset (Only USA & Canada) On Screen 1: 10sec / 2: 30sec / 3: Always XM Channel Number Upper 4 bit (Only USA & Canada) XM Channel Number Lower 4 bit (Only USA & Canada) LFE Level SP Upper 4 bit LFE Level SP Lower 4 bit LFE Level HP Upper 4 bit LFE Level HP Lower 4 bit Manual Audio Delay Upper 4 bit Manual Audio Delay Lower 4 bit Initial Volume Upper 4 bit added to RX-V18 Initial Volume Lower 4 bit added to RX-V18 Max Volume 0: -30dB / 1: -25dB / 2: -20dB / 3: -15dB / 4: -10dB / 5: -5dB / 6: 0dB / 7: 5dB / 8: 10dB / 9: 15dB / A: 16.5dB added to RX-V18 Decoder Mode Set 0: Auto / 1: Last Audio Select Set 0: Auto / 1: Last Dimmer 0: -4 / 1: -3 / 2: -2 / 3: -1 / 4: 0 Don't care |
| --- | --- | --- | --- |
| DT63 |  |  |  |
| DT64 | 48 |  |  |
| DT65 |  |  |  |
| DT66 | 49 |  |  |
| DT67 |  |  |  |
| DT68 | 90 |  |  |
| DT69 | 91 |  |  |
| DT70 | 92 |  |  |
| DT71 | 93 |  |  |
| DT72 | 94 |  |  |
| DT73 |  |  |  |
| DT74 | 51 |  |  |
| DT75 |  |  |  |
| DT76 | 52 |  |  |
| DT77 |  |  |  |
| DT78 | 53 |  |  |
| DT79 |  |  |  |
| DT80 | 5B |  |  |
| DT81 |  |  |  |
| DT82 | 5A |  |  |
| DT83 | 5F |  |  |
| DT84 | 60 |  |  |
| DT85 | 61 |  |  |
| DT86 |  |  |  |
| DT87 | 62 | 0 - A OSD Shift / GUI position Upper 4 bit OSD Shift / GUI position Lower 4 bit Gray back 0: Off / 1: Auto (Only RX-V1800) Video conversion 0: Off / 1: On D. Rang 0 - A 0 / 1 0 / 1 |  |
| DT88 |  |  |  |
| DT89 | 63 |  |  |
| DT90 | 69 |  |  |
| DT91 | 64 | 0 - 2 0 - 2 0 / 1 0 / 1 0 / 1 0 - 2 0 / 1 0 - 2 0 - 4 0 - 3 0 / 1 0 - 2 0 / 1 0 / 1 0 - 2 0 / 1 0 - F | SP 0: MAX / 1: STD / 2: MIN D. Rang HP 0: MAX / 1: STD / 2: MIN Zone2 Volume Out 0: Variable / 1: Fixed Zone3 Volume Out 0: Variable / 1: Fixed Memory guard 0: Off / 1: On SP set Center 0: Large / 1: Small / 2: None Front 0: Large / 1: Small Sur. L/R 0: Large / 1: Small / 2: None Sur. Back 0: Large x2 / 1: Large x1 / 2:Small x2 / 3:Small x1 / 4:None Zone3 Amplifier 0: EXT / 1: INT: Speaker[1] / 2: INT: Speaker[2] / 3: INT Both SP set Presence 0: Yes / 1: None LFE/BASS 0: SWFR / 1: Front / 2: Both Subwoofer Phase Upper 4 bit : Phase 0:Normal / 1:Reverse Subwoofer Phase Lower 4 bit : Don't Care Test mode 0: OFF / 1: ON EQ Select 0: Auto PEQ / 1: GEQ / 2: Off (Only RX-V1800) Wall Paper 0: Yes / E: Gray / F: None (Only RX-V3800) Don't care Don't care Don't care HDMI Support Audio 0: RX-V3800 or RX-V1800 or HTR-6190 / 1: Other |
| DT92 | 65 |  |  |
| DT93 | 66 |  |  |
| DT94 | 6B |  |  |
| DT95 | 68 |  |  |
| DT96 | 70 |  |  |
| DT97 | 71 |  |  |
| DT98 | 72 |  |  |
| DT99 | 73 |  |  |
| DT100 | 3F |  |  |
| DT101 | 74 |  |  |
| DT102 | 75 |  |  |
| DT103 | 76 |  |  |
| DT104 |  |  |  |
| DT105 | 80 |  |  |
| DT106 | A7 |  |  |
| DT107 | 58 |  |  |
| DT108 |  |  |  |
| DT109 |  |  |  |
| DT110 |  |  |  |
| DT111 | 8F |  |  |
| DT112 | 85 | 0 / 1 Component I/P 0: Off / 1: On |  |
| DT113 | 86 | 0 / 1 HDMI IP 0: Off / 1: On (Only RX-V1800) Deletion |  |
| DT114 | B6 | 0 - 5 0 - 4 | GUI Language 0: English / 1: Japanese / 2: French / 3: German / 4: Spanish / 5: Russian (Only RX- HDMI Up-Scaling 0: Through / 1: 480p(576p) / 2: 1080i / 3: 720p / 4: 1080p added to RX-V18 |
| DT115 | 87 |  |  |
| DT116 | 88 | 0 - 2 HDMI Aspect 0: Through / 1: 16: 9 Normal / 2: Smart Zoom added to RX-V18 0 - 2 THX SB Speaker Distance 0: undef 1ft(0.3m) / 1: 1-4ft (0.3-1.2m) / 2: over 4ft (1.2m) |  |
| DT117 | 8A |  |  |
| DT118 | 6C | 0 - 2 0 - 9 0 / 1 0 / 1 0 / 1 | Zone OSD 0: Off / 1: Zone2 / 2: Zone2 & Zone3 (Only RX-V3800) Decoder Select 0: Pro Logic / 1: PLIIx Movie / 2: PLIIx Music / 3: PLIIx Game / 4: Neo: 6 Cinema / 5: Neo: 6 Music / 6: CSII Cinema* / 7: CSII Music* / 9: Neural Sur. *Only Japanese Remote ID Tuner 0: ID1 / 1: ID2 Advanced Setup 0: Off / 1: On Remote ID AMP 0: ID1 / 1: ID2 |
| DT119 | 6E |  |  |
| DT120 | B5 |  |  |
| DT121 | B0 |  |  |
| DT122 | B1 |  |  |
| DT123 | B2 | 0 / 1 Fan Control Mode 0: Auto / 1: Cont |  |
| DT124 | B3 | 0 / 1 0 / 1 0 / 1 0 - E | Speaker Impedance 0: 8 ohm / 1: 6 ohm (Except Japanese Model) Tuner Setup 0: AM10/FM100 / 1: AM9/FM50 (Only Destination R & L) Pure Direct 0: Off / 1: On Zone3 Input 0: PHONO / 1: CD / 2: TUNER / 3: CD-R / 4: MD/TAPE / 5: DVD / 6: DTV/CBL 9: VCR / A: DVR / C: V-AUX / D: NET/USB* / E:XM** / F: BD/HD DVD *Only RX-V3800 **Only USA & Canada |
| DT125 | B4 |  |  |
| DT126 | 8C |  |  |
| DT127 | A0 |  |  |

6

00 too 00 too

00 too

V3800) 00 too 00 too

Model

| DT128 | A1 |
| --- | --- |
| DT129 | A2 |
| DT130 |  |
| DT131 | B9 |
| DT132 | 7B |
| DT133 | BA |
| DT134 | BB |
| DT135 | 7E |
| DT136 | BC |
| DT137 | 7D |
| DT138 |  |
| DT139 | 4B |
| DT140 |  |
| DT141 | 4C |
| DT142 |  |
| DT143 | A8 |
| DT144 | BD |
| DT145 | 14 |
| DT146 |  |
| DT147 | 15 |
| DT148 |  |
| DT149 | 67 |
| DT150 | 6F |
| DT151 | 96 |
| DT152 | 97 |
| DT153 | 98 |
| DT154 | 99 |
| DT155 | 9A |
| DT156 | A9 |
| DT157 | AB |
| DT158 |  |
| DT159 | AD |
| DT160 |  |
| DT161 | AA |
| DT162 | AC |
| DT163 |  |
| DT164 | AE |
| DT165 |  |
| DT166 | 9B |
| DT167 | BF |
| DT168 | 4D |
| DT169 |  |
| DT170 | 4E |
| DT171 |  |
| DT172 | 16 |
| DT173 |  |

0 / 1 0 - F

0 - F 0 / 1 0 - C

0 / 1

0 / 1 0 - 8

0 / 1 0 / 1

0 - F 0 - F

0 - F 0 - F

0 / 1 0 / 1

0 - F 0 - F

0 - F

0 - F

0 - 1 0 - C

0 - 1

0 - 2

0 - 2 0 - 2

0 - 1 0 - A

RX-Vx800 RS-232C Protocol

Zone3 Mute	0: Off / 1:On

Zone3 Volume	Upper 4 bit

Lower 4 bit 
Remote Sensor	0: On / 1: Off

Multi CH Select	0: 6ch / 2: 8ch CD / 3: 8ch CD-R / 4: 8ch MD/TAPE / 5: 8ch DVD / 6: 8ch DTV/CBL /

9: 8ch VCR / A: 8ch DVR / C: 8ch V-AUX /F: 8ch BD/HD DVD 
Remote ID XM	0: ID1 / 1: ID2  (Only USA & Canada)

Bi – AMP	0: On / 1: Off

Subwoofer Crossover	0: 40Hz / 1: 60Hz / 2: 80Hz / 3: 90Hz / 4: 100Hz / 5: 110Hz / 6: 120Hz /

7: 160Hz / 8: 200Hz

TV Format	0: PAL / 1: NTSC  (Only RX-V3800)

PR/SB Priority	0: Presence / 1: Surround Back

Don't care

Zone2 Tone Control Bass	Upper 4 bit Lower 4 bit 
Zone2 Tone Control Treble	Upper 4 bit Lower 4 bit

Tone Bypass	0: Auto / 1: Off

Wake on RS-232C Access	0: No / 1: Yes 
Bit Rate	Upper 4 bit

Bit Rate	Lower 4 bit

Dialog	Upper 4 bit

Dialog	Lower 4 bit

FL Scroll	0: Continue / 1: Once

Multi CH BGV	0: Off / 1: Last / 5: DVD / 6: DTV/CBL / 9: VCR / A: DVR / C: V-AUX / F: BD/HD DVD iPod Charge on Standby	0: Off / 1: Auto

iPod Repeat	0: Off / 1: One / 2: All

iPod Shuffle	0: Off / 1: Songs / 2: Albums

NET/USB Repeat	0: Off / 1: Single / 2: All  (Only RX-V3800)

NET/USB Shuffle	0: Off / 1: On  (Only RX-V3800)

Zone2 Max Volume	0: -30dB / 1: -25dB / 2: -20dB / 3: -15dB / 4: -10dB / 5: -5dB / 6: 0dB / 7: 5dB /

*OSD = On Screen Display

0 - F

0 - F

0 - F

0 - F

0 - A

0 - F

0 - F

0 - F

0 - F

0 - 2

0 - 1

0 - F 0 - F 0 - F 0 - F

0 - F

0 - F

8: 10dB / 9: 15dB / A: 16.5dB	added to RX-V18

Zone2 Initial Volume	Upper 4 bit	added to RX-V18

Zone2 Initial Volume	Lower 4 bit	added to RX-V18

Zone2 Balance	Upper 4 bit

Zone2 Balance	Lower 4 bit

Zone3 Max Volume	0: -30dB / 1: -25dB / 2: -20dB / 3: -15dB / 4: -10dB / 5: -5dB / 6: 0dB / 7: 5dB /

8: 10dB / 9: 15dB / A: 16.5dB	added to RX-V18

Zone3 Initial Volume	Upper 4 bit	added to RX-V18

Zone3 Initial Volume	Lower 4 bit	added to RX-V18

Zone3 Balance	Upper 4 bit

Zone3 Balance	Lower 4 bit

NET/USB Source Select	0: PC/MCX / 1: NET Radio / 2: USB  (Only RX-V3800)

Monitor Check	0: Skip / 1: Yes	added to RX-V18

Zone3 Tone Control Bass	Upper 4 bit Lower 4 bit 
Zone3 Tone Control Treble	Upper 4 bit Lower 4 bit 
Flag	Upper 4 bit

Lower 4 bit

7

00 too 00 too 00 too

00 too 00 too 00 too

00 too

3.

Control Commands

Control Command

RX-Vx800

Report Command

RX-Vx800 RS-232C Protocol

Host

* The RX-Vx800 can receive control commands only when the power is on (except Power commands and System commands*).

* Do not send any control commands when the system status is Wait. No commands are permitted until the RX-Vx800 reports OK.

* The RX-Vx800 will send a Report Command** within 1 sec of receiving the Control Command. If no Report Command is received, resend control

command (max 5 times).

If the RX-Vx800 doesn't send a Report Command after the fifth retry, cancel the transaction because there may be a problem.

* 'SW' switches the type of the control command. When the 'SW' is set to '0', you can control the RX-Vx800 remotely via RS-232C.

* The RX-Vx800 will only send one report command for each type of control. The Report Command will report only the final status of all settings in

strings of commands (may not report all steps in a status, only final status).

For example, if the user sets the input selector on the unit to DTV just after the host sends a command to change the input to CD, the RX-Vx800 may   report only the final status that the input was changed to DTV by the system operation.

*System command, **Report command --> described in later

- Command Switch ( changing the type of control command )

Control command

STX

SW

CMDT0

CMDT1

CMDT2

CMDT3

ETX

SW=0	Operation Command

(Remote Control) 
SW=1	Command

4

byte remot

e control c

ode

( Not in use ) 
SW=2	System command

SW=3	Packet data

command

comm

and

data

da

data

ta

| function name | function | data (ASCII) | range (HEX) |
| --- | --- | --- | --- |
| SW | command switch | 0 - 9 | 0 - 9 |
| CMDT0 - 3 | command & data | 0 - 9, A - F | variable |

* 'SW' switches the command type of the Control Command.

SW=0

: 4 byte command for remote control code ( CMDT0 = '7' )

SW=1

SW=2

: 1 byte command

: 2 byte command

0 - F (HEX expression in ASCII)

10 - FF (HEX expression in ASCII)

SW=3

: 4 byte packet data for test data transmission

Ext. Control command

STX

SW

CMDT0

CMDT1

CMDT2

CMDT3

CMDT4

CMDT5

CMDT6

CMDT7

CMDT8

ETX

SW=0	Extended

Operation Command	9 byte remote control code

(Remote Control)

| function name | function | data (ASCII) | range (HEX) |
| --- | --- | --- | --- |
| SW | command switch | 0 | 0 |
| CMDT0 | command | F | F |
| CMDT1 - 8 | command | 0 - 9, A - F | variable |

* 'SW' switches the command type of the Control Command.

SW=0  : 9 byte command for remote control code ( CMDT0 = 'F' )

8

RX-Vx800 RS-232C Protocol

3.1  System Commands ( SW = '2' )

Command	Data

STX	SW ( '2' )	CMDT0	CMDT1	CMDT2	CMDT3	ETX

System Commands can be made by setting the 'SW' byte in the Control Command to '2'. With System commands you can control the RX-Vx800's system settings (Report Command Enable / Disable, Report Command delay, etc...)

With a System Command you can also...

- set absolute master volume value.

- send text strings to the On Screen Display (OSD).

- request RX-Vx800 text data regarding tuner freq., master volume, input name, zone 2 input name.

| SW |  |  | Command |  | Data |  | Report command |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CMDT0 | CMDT1 | Function | CMDT2 | CMDT3 | Parameter | Type | RCMD1, | RDAT1, |  |
| 2 | 0 | 0 | Report Command Code | 0 | 0 | Enable | 0 | 00 | 00(OK) |  |
|  |  |  |  | 0 | 1 | Disable |  |  |  |  |
| 2 | 0 | 1 | Time between two report commands (Report Command Delay) | 0 | 0 | real time | 0 | 00 | 00(OK) |  |
|  |  |  |  | 0 | 1 | 50ms |  |  |  |  |
|  |  |  |  | 0 | 2 | 100ms |  |  |  |  |
|  |  |  |  | 0 | 3 | 150ms |  |  |  |  |
|  |  |  |  | 0 | 4 | 200ms |  |  |  |  |
|  |  |  |  | 0 | 5 | 250ms |  |  |  |  |
|  |  |  |  | 0 | 6 | 300ms |  |  |  |  |
|  |  |  |  | 0 | 7 | 350ms |  |  |  |  |
|  |  |  |  | 0 | 8 | 400ms |  |  |  |  |
| 2 | 1 | 0 | OSD message start command | 0 | 0 | Start | 0 | 00 | 00(OK) |  |
| 2 | 2 | 0 | Tuning frequency text request Main volume value text request Zone2 volume value text request Main Input name text request Zone2 input name text request Zone3 volume value text request Zone3 input name text request | 0 | 0 |  | Refer to the following section |  |  |  |
|  |  |  |  | 0 | 1 |  |  |  |  |  |
|  |  |  |  | 0 | 2 |  |  |  |  |  |
|  |  |  |  | 0 | 3 |  |  |  |  |  |
|  |  |  |  | 0 | 4 |  |  |  |  |  |
|  |  |  |  | 0 | 5 |  |  |  |  |  |
|  |  |  |  | 0 | 6 |  |  |  |  |  |
| 2 | 2 | F | Firmware version request | 0 | 0 |  | 0 | 00 |  |  |
| 2 | 3 | 0 | Main volume direct setting | X | X |  | 0 | 26 |  |  |
| 2 | 3 | 1 | Zone2 volume direct setting | X | X |  | 0 | 27 |  |  |
| 2 | 3 | 4 | Zone3 volume direct setting | X | X |  | 0 | A2 |  |  |
| 2 | 3 | 8 | Mute Type | 0 | 0 | Full | 0 | A5 |  |  |
|  |  |  |  | 0 | 1 | -20dB |  |  |  |  |
| 2 | 3 | E | Zone2 Amplifier | 0 | 0 | EXT | 0 | 3E |  |  |
|  |  |  |  | 0 | 1 | INT: Speaker[1] |  |  |  |  |
|  |  |  |  | 0 | 2 | INT: Speaker[2] |  |  |  |  |
|  |  |  |  | 0 | 3 | INT: Both |  |  |  |  |
| 2 | 3 | F | Zone3 Amplifier | 0 | 0 | EXT | 0 | 3F |  |  |
|  |  |  |  | 0 | 1 | INT: Speaker[1] |  |  |  |  |
|  |  |  |  | 0 | 2 | INT: Speaker[2] |  |  |  |  |
|  |  |  |  | 0 | 3 | INT: Both |  |  |  |  |
| 2 | 4 | 0 | Level Front R | X | X |  | 0 | 40 |  |  |
| 2 | 4 | 1 | Level Front L | X | X |  | 0 | 41 |  |  |
| 2 | 4 | 2 | Level Center | X | X |  | 0 | 42 |  |  |
| 2 | 4 | 3 | Level Surround R | X | X |  | 0 | 43 |  |  |
| 2 | 4 | 4 | Level Surround L | X | X |  | 0 | 44 |  |  |
| 2 | 4 | 5 | Level Presence R | X | X |  | 0 | 47 |  |  |
| 2 | 4 | 6 | Level Presence L | X | X |  | 0 | 48 |  |  |
| 2 | 4 | 7 | Level Surround Back R | X | X |  | 0 | 45 |  |  |
| 2 | 4 | 8 | Level Surround Back L | X | X |  | 0 | 46 |  |  |
| 2 | 4 | 9 | Level Subwoofer (1) | X | X |  | 0 | 49 |  |  |
| 2 | 4 | A | Level Subwoofer (2) | X | X |  | 0 | 4A |  |  |
| 2 | 4 | B | Zone2 Tone Control Bass | X | X |  | 0 | 4B |  |  |
| 2 | 4 | C | Zone2 Tone Control Treble | X | X |  | 0 | 4C |  |  |
| 2 | 4 | D | Zone3 Tone Control Bass | X | X |  | 0 | 4D |  |  |
| 2 | 4 | E | Zone3 Tone Control Treble | X | X |  | 0 | 4E |  |  |
| 2 | 5 | 1 | LFE Level for Speaker | X | X |  | 0 | 51 |  |  |
| 2 | 5 | 2 | LFE Level for Headphone | X | X |  | 0 | 52 |  |  |
| 2 | 5 | 3 | Manual Audio Delay | X | X |  | 0 | 53 |  |  |
| 2 | 5 | 6 | HDMI Auto Audio Delay |  |  |  | 0 | 56 |  |  |
| 2 | 5 | 7 | DSP 3D | 0 | 0 | Off | 0 | 57 |  |  |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | 5 | 8 | Wall Paper | 0 | 0 | Yes | 0 | 58 |  | Only RX-V3800 |
|  |  |  |  | 0 | E | Gray |  |  |  |  |
|  |  |  |  | 0 | F | None |  |  |  |  |
| 2 | 5 | 9 | HDMI Auto Lipsync | 0 | 0 | Auto | 0 | 59 |  |  |
|  |  |  |  | 0 | 1 | Off |  |  |  |  |

9

o

RX-Vx800 RS-232C Protocol

| SW |  |  | Command |  | Data |  | Report command |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CMDT0 | CMDT1 | Function | CMDT2 | CMDT3 | Parameter | Type | RCMD1, | RDAT1, |  |
| 2 | 5 | A | MAX Volume | X | X |  | 0 | 5A |  | added to RX-V1800 |
| 2 | 5 | B | Initial Volume | X | X |  | 0 | 5B |  | added to RX-V1800 |
| 2 | 5 | C | Extended Surround Set | 0 | 0 | Auto | 0 | 5C |  |  |
|  |  |  |  | 0 | 1 | Last |  |  |  |  |
| 2 | 5 | D | Adaptive DSP Level | 0 | 0 | Auto | 0 | 5D |  |  |
|  |  |  |  | 0 | 1 | Off |  |  |  |  |
| 2 | 5 | E | Decoder Mode | X | X |  | 0 | 5E |  |  |
| 2 | 5 | F | Decoder Mode Set | 0 | 0 | Auto | 0 | 5F |  |  |
|  |  |  |  | 0 | 1 | Last |  |  |  |  |
| 2 | 6 | 0 | Audio Select Set | 0 | 0 | Auto | 0 | 60 |  |  |
|  |  |  |  | 0 | 1 | Last |  |  |  |  |
| 2 | 6 | 1 | Dimmer | 0 | 0 | -4 | 0 | 61 |  |  |
|  |  |  |  | 0 | 1 | -3 |  |  |  |  |
|  |  |  |  | 0 | 2 | -2 |  |  |  |  |
|  |  |  |  | 0 | 3 | -1 |  |  |  |  |
|  |  |  |  | 0 | 4 | 0 |  |  |  |  |
| 2 | 6 | 2 | OSD Shift* / GUI Position** | X | X |  | 0 | 62 |  | *RX-V1800 **RX-V3800 |
| 2 | 6 | 3 | Gray Back | 0 | 0 | Off | 0 | 63 |  | Only RX-V1800 |
|  |  |  |  | 0 | 1 | Auto |  |  |  |  |
| 2 | 6 | 4 | Dynamic Range for Speaker | 0 | 0 | MAX | 0 | 64 |  |  |
|  |  |  |  | 0 | 1 | STD |  |  |  |  |
|  |  |  |  | 0 | 2 | MIN |  |  |  |  |
| 2 | 6 | 5 | Dynamic Range for Headphone | 0 | 0 | MAX | 0 | 65 |  |  |
|  |  |  |  | 0 | 1 | STD |  |  |  |  |
|  |  |  |  | 0 | 2 | MIN |  |  |  |  |
| 2 | 6 | 6 | Zone2 Volume Out | 0 | 0 | Variable | 0 | 66 |  |  |
|  |  |  |  | 0 | 1 | Fixed |  |  |  |  |
| 2 | 6 | 7 | FL Scroll | 0 | 0 | Continue | 0 | 67 |  |  |
|  |  |  |  | 0 | 1 | Once |  |  |  |  |
| 2 | 6 | 8 | Memory Guard | 0 | 0 | Off | 0 | 68 |  |  |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | 6 | 9 | Video Conversion | 0 | 0 | Off | 0 | 69 |  |  |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | 6 | B | Zone3 Volume Out | 0 | 0 | Variable | 0 | 6B |  |  |
|  |  |  |  | 0 | 1 | Fixed |  |  |  |  |
| 2 | 6 | C | Zone OSD | 0 | 0 | Off | 0 | 6C |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | Zone2 |  |  |  |  |
|  |  |  |  | 0 | 2 | Zone2 & 3 |  |  |  |  |
| 2 | 6 | D | Adaptive DRC | 0 | 0 | Auto | 0 | 6D |  |  |
|  |  |  |  | 0 | 1 | Last |  |  |  |  |
| 2 | 6 | F | Multi CH BGV | 0 | 0 | Off |  |  |  | Deletion |
|  |  |  |  | 0 | 1 | Last |  |  |  |  |
|  |  |  |  | 0 | 5 | DVD |  |  |  |  |
|  |  |  |  | 0 | 6 | DTV/CBL |  |  |  |  |
|  |  |  |  | 0 | 7 | CBL/SAT |  |  |  |  |
|  |  |  |  | 0 | 9 | VCR |  |  |  |  |
|  |  |  |  | 0 | A | DVR |  |  |  |  |
|  |  |  |  | 0 | C | V-AUX |  |  |  |  |
|  |  |  |  | 0 | F | BD/HD DVD |  |  |  |  |
| 2 | 7 | 0 | Speaker Set Center | 0 | 0 | Large | 0 | 70 |  |  |
|  |  |  |  | 0 | 1 | Small |  |  |  |  |
|  |  |  |  | 0 | 2 | None |  |  |  |  |
| 2 | 7 | 1 | Speaker Set Front | 0 | 0 | Large | 0 | 71 |  |  |
|  |  |  |  | 0 | 1 | Small |  |  |  |  |
| 2 | 7 | 2 | Speaker Set Surround | 0 | 0 | Large | 0 | 72 |  |  |
|  |  |  |  | 0 | 1 | Small |  |  |  |  |
|  |  |  |  | 0 | 2 | None |  |  |  |  |
| 2 | 7 | 3 | Speaker Set Surround Back | 0 | 0 | Large x2 | 0 | 73 |  |  |
|  |  |  |  | 0 | 1 | Large x1 |  |  |  |  |
|  |  |  |  | 0 | 2 | Small x2 |  |  |  |  |
|  |  |  |  | 0 | 3 | Small x1 |  |  |  |  |
|  |  |  |  | 0 | 4 | None |  |  |  |  |
| 2 | 7 | 4 | Speaker Set Presence | 0 | 0 | Yes | 0 | 74 |  |  |
|  |  |  |  | 0 | 1 | None |  |  |  |  |
| 2 | 7 | 5 | Speaker Set Bass Out | 0 | 0 | Subwoofer | 0 | 75 |  |  |
|  |  |  |  | 0 | 1 | Front |  |  |  |  |
|  |  |  |  | 0 | 2 | Both |  |  |  |  |
| 2 | 7 | 6 | Speaker Set Subwoofer Phase | 0 | X | Normal | 0 | 76 |  | Lower 4 bit: Don't care |
|  |  |  |  | 1 | X | Reverse |  |  |  |  |

10

RX-Vx800 RS-232C Protocol

| SW |  |  | Command |  | Data |  | Report command |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CMDT0 | CMDT1 | Function | CMDT2 | CMDT3 | Parameter | Type | RCMD1, | RDAT1, |  |
| 2 | 7 | B | Multi CH Select | 0 | 0 | 6ch | 0 | 7B |  | Deletion |
|  |  |  |  | 0 | 2 | 8ch CD |  |  |  |  |
|  |  |  |  | 0 | 3 | 8ch CD-R |  |  |  |  |
|  |  |  |  | 0 | 4 | 8ch MD/TAPE |  |  |  |  |
|  |  |  |  | 0 | 5 | 8ch DVD |  |  |  |  |
|  |  |  |  | 0 | 6 | 8ch DTV/CBL |  |  |  |  |
|  |  |  |  | 0 | 7 | 8ch CBL/SAT |  |  |  |  |
|  |  |  |  | 0 | 9 | 8ch VCR |  |  |  |  |
|  |  |  |  | 0 | A | 8ch DVR |  |  |  |  |
|  |  |  |  | 0 | C | 8ch V-AUX |  |  |  |  |
|  |  |  |  | 0 | F | 8ch BD/HD DVD |  |  |  |  |
| 2 | 7 | D | PR/SB Priority | 0 | 0 | Presence | 0 | 7D |  |  |
|  |  |  |  | 0 | 1 | Sur. Back |  |  |  |  |
| 2 | 7 | E | Subwoofer Crossover | 0 | 0 | 40Hz | 0 | 7E |  |  |
|  |  |  |  | 0 | 1 | 60Hz |  |  |  |  |
|  |  |  |  | 0 | 2 | 80Hz |  |  |  |  |
|  |  |  |  | 0 | 3 | 90Hz |  |  |  |  |
|  |  |  |  | 0 | 4 | 100Hz |  |  |  |  |
|  |  |  |  | 0 | 5 | 110Hz |  |  |  |  |
|  |  |  |  | 0 | 6 | 120Hz |  |  |  |  |
|  |  |  |  | 0 | 7 | 160Hz |  |  |  |  |
|  |  |  |  | 0 | 8 | 200Hz |  |  |  |  |
| 2 | 8 | 0 | Test Tone | 0 | 0 | Off | 0 | 80 |  |  |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | 8 | 5 | Component I/P | 0 | 0 | Off | 0 | 85 |  |  |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | 8 | 6 | HDMI I/P | 0 | 0 | Off | 0 | 86 |  | Deletion |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | 8 | 7 | HDMI Up-Scaling | 0 | 0 | Through | 0 | 87 |  | added to RX-V1800 |
|  |  |  |  | 0 | 1 | 480p(576p) |  |  |  |  |
|  |  |  |  | 0 | 2 | 1080i |  |  |  |  |
|  |  |  |  | 0 | 3 | 720p |  |  |  |  |
|  |  |  |  | 0 | 4 | 1080p |  |  |  |  |
| 2 | 8 | 8 | HDMI Aspect | 0 | 0 | Through | 0 | 88 |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | 16:9 Normal |  |  |  |  |
|  |  |  |  | 0 | 2 | Smart Zoom |  |  |  |  |
| 2 | 8 | A | THX SB Speaker Dist. | 0 | 0 | under 1ft (0.3m) | 0 | 8A |  |  |
|  |  |  |  | 0 | 1 | 1-4ft (0.3-1.2m) |  |  |  |  |
|  |  |  |  | 0 | 2 | over 4ft (1.2m) |  |  |  |  |
| 2 | 8 | B | Night Mode | 0 | 0 | Off | 0 | 8B |  | Deletion |
|  |  |  |  | 1 | 0 | Cinema Low |  |  |  |  |
|  |  |  |  | 1 | 1 | Cinema Mid |  |  |  |  |
|  |  |  |  | 1 | 2 | Cinema High |  |  |  |  |
|  |  |  |  | 2 | 0 | Music Low |  |  |  |  |
|  |  |  |  | 2 | 1 | Music Mid |  |  |  |  |
|  |  |  |  | 2 | 2 | Music High |  |  |  |  |
| 2 | 8 | F | HDMI Support Audio | 0 | 0 | RX-V1800 | 0 | 8F |  |  |
|  |  |  |  |  |  | RX-V3800 |  |  |  |  |
|  |  |  |  | 0 | 1 | Other |  |  |  |  |
| 2 | 9 | 3 | On Screen | 0 | 0 | Off | 0 | 93 |  |  |
|  |  |  |  | 0 | 1 | 10sec |  |  |  |  |
|  |  |  |  | 0 | 2 | 30sec |  |  |  |  |
|  |  |  |  | 0 | 3 | Always |  |  |  |  |
| 2 | 9 | 4 | XM Channel Number | X | X |  | 0 | 94 |  | Only USA & Canada |
| 2 | 9 | 5 | Hold / Release XM Display | 0 | 0 | Release | 0 | 95 |  | Only USA & Canada |
|  |  |  |  | 0 | 1 | Hold |  |  |  |  |
| 2 | 9 | 6 | iPod Charge on Standby | 0 | 0 | Off | 0 | 96 |  |  |
|  |  |  |  | 0 | 1 | Auto |  |  |  |  |
| 2 | 9 | 7 | iPod Repeat | 0 | 0 | Off | 0 | 97 |  |  |
|  |  |  |  | 0 | 1 | One |  |  |  |  |
|  |  |  |  | 0 | 2 | All |  |  |  |  |
| 2 | 9 | 8 | iPod Shuffle | 0 | 0 | Off | 0 | 98 |  |  |
|  |  |  |  | 0 | 1 | Songs |  |  |  |  |
|  |  |  |  | 0 | 2 | Albums |  |  |  |  |
| 2 | 9 | 9 | NET/USB Repeat | 0 | 0 | Off | 0 | 99 |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | Single |  |  |  |  |
|  |  |  |  | 0 | 2 | All |  |  |  |  |
| 2 | 9 | A | NET/USB Shuffle | 0 | 0 | Off | 0 | 9A |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | On |  |  |  |  |

11

RX-Vx800 RS-232C Protocol

| SW |  |  | Command |  | Data |  | Report command |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CMDT0 | CMDT1 | Function | CMDT2 | CMDT3 | Parameter | Type | RCMD1, | RDAT1, |  |
| 2 | 9 | C | NET/USB Preset Recall | X | X |  | 0 | 9C |  | Only RX-V3800 |
| 2 | 9 | D | NET/USB Preset Memory | X | X |  | 0 | 9D |  | Only RX-V3800 |
| 2 | A | 7 | EQ (Equalizer) Select | 0 | 0 | Auto PEQ | 0 | A7 |  | Only RX-V1800 |
|  |  |  |  | 0 | 1 | GEQ |  |  |  |  |
|  |  |  |  | 0 | 2 | EQ Off |  |  |  |  |
| 2 | A | 8 | Tone Control Auto Bypass | 0 | 0 | Auto | 0 | A8 |  |  |
|  |  |  |  | 0 | 1 | Off |  |  |  |  |
| 2 | A | 9 | Zone2 MAX Volume | X | X |  | 0 | A9 |  | added to RX-V1800 |
| 2 | A | A | Zone3 MAX Volume | X | X |  | 0 | AA |  | added to RX-V1800 |
| 2 | A | B | Zone2 Initial Volume | X | X |  | 0 | AB |  | added to RX-V1800 |
| 2 | A | C | Zone3 Initial Volume | X | X |  | 0 | AC |  | added to RX-V1800 |
| 2 | A | D | Zone2 Balance | X | X |  | 0 | AD |  |  |
| 2 | A | E | Zone3 Balance | X | X |  | 0 | AE |  |  |
| 2 | A | F | Music Enhancer | 0 | 0 | Off | 0 | 28 |  | Deletion |
|  |  |  |  | 0 | 1 | 2ch Low |  |  |  |  |
|  |  |  |  | 0 | 2 | 2ch High |  |  |  |  |
|  |  |  |  | 0 | 3 | 7ch Low |  |  |  |  |
|  |  |  |  | 0 | 4 | 7ch High |  |  |  |  |
| 2 | B | 0 | Advanced Setup | 0 | 0 | Off | 0 | B0 |  | Effective in STANDBY |
|  |  |  |  | 0 | 1 | On |  |  |  |  |
| 2 | B | 1 | Remote ID for AMP | 0 | 0 | ID1 | 0 | B1 |  |  |
|  |  |  |  | 0 | 1 | ID2 |  |  |  |  |
| 2 | B | 2 | Fan Control Mode | 0 | 0 | Auto | 0 | B2 |  |  |
|  |  |  |  | 0 | 1 | Cont. |  |  |  |  |
| 2 | B | 3 | Speaker Impedance | 0 | 0 | 8 ohm | 0 | B3 |  | xcept Japanese Mod |
|  |  |  |  | 0 | 1 | 6 ohm |  |  |  |  |
| 2 | B | 4 | Tuner Frequency Step | 0 | 0 | 10k/100kHz | 0 | B4 |  | Only Destination R & |
|  |  |  |  | 0 | 1 | 9k/50kHz |  |  |  |  |
| 2 | B | 5 | Remote ID for Tuner | 0 | 0 | ID1 | 0 | B5 |  |  |
|  |  |  |  | 0 | 1 | ID2 |  |  |  |  |
| 2 | B | 6 | Language | 0 | 0 | English | 0 | B6 |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | Japanese |  |  |  |  |
|  |  |  |  | 0 | 2 | French |  |  |  |  |
|  |  |  |  | 0 | 3 | German |  |  |  |  |
|  |  |  |  | 0 | 4 | Spanish |  |  |  |  |
|  |  |  |  | 0 | 5 | Russian |  |  |  |  |
| 2 | B | 7 | User Preset | 0 | 0 | Cancel | 0 | B7 |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | All |  |  |  |  |
|  |  |  |  | 0 | 2 | DSP Parameter |  |  |  |  |
|  |  |  |  | 0 | 3 | Video |  |  |  |  |
|  |  |  |  | 0 | 4 | Network |  |  |  |  |
| 2 | B | 8 | Video Reset | 0 | 0 | Cancel | 0 | B8 |  | Deletion |
|  |  |  |  | 0 | 1 | Yes |  |  |  |  |
| 2 | B | 9 | Remote Sensor | 0 | 0 | On | 0 | B9 |  |  |
|  |  |  |  | 0 | 1 | Off |  |  |  |  |
| 2 | B | A | Remote ID for XM | 0 | 0 | ID1 | 0 | BA |  | Only USA & Canada |
|  |  |  |  | 0 | 1 | ID2 |  |  |  |  |
| 2 | B | B | Bi-Amplifier | 0 | 0 | On | 0 | BB |  |  |
|  |  |  |  | 0 | 1 | Off |  |  |  |  |
| 2 | B | C | TV Format | 0 | 0 | PAL | 0 | BC |  | Only RX-V3800 |
|  |  |  |  | 0 | 1 | NTSC |  |  |  |  |
| 2 | B | D | Wake on RS-232C Access | 0 | 0 | No | 0 | BD |  |  |
|  |  |  |  | 0 | 1 | Yes |  |  |  |  |
| 2 | B | E | Network Reset | 0 | 0 | Cancel | 0 | BE |  | Deletion |
|  |  |  |  | 0 | 1 | Yes |  |  |  |  |
| 2 | B | F | Monitor Check | 0 | 0 | Skip | 0 | BF |  | added to RX-V1800 |
|  |  |  |  | 0 | 1 | Yes |  |  |  |  |

12

RX-Vx800 RS-232C Protocol

*OSD message function

The OSD Message function can display a message of 16 characters to the RX-Vx800's OSD for a few seconds.

The command sequence block will start by sending "start command" as mentioned above, followed by four bytes of packet data (SW:3) repeated four times. Then a message of sixteen characters (ASCII) will display and the command block will finish automatically.

(ex.) Want to display " Test message ! " characters to OSD.

1.

Send the start command.

STX

2	1	0	0	0	ETX

2.

Send SW:3 commands four times as follows.

| STX | 3 | ' ' | 'T' | 'e' | 's' | ETX |
| --- | --- | --- | --- | --- | --- | --- |
| STX | 3 | 't' | ' ' | 'm' | 'e' | ETX |
| STX | 3 | 's' | 's' | 'a' | 'g' | ETX |
| STX | 3 | 'e' | ' ' | '!' | ' ' | ETX |

3.	The command block will be finished automatically.

Characters available for displaying the message are as follows:

" "(SPACE) "!" "#" "%" "&" "(" ")" "*" "+" "," "-" "." "0" "1" "2" "3" "4" "5" "6" "7" "8" "9" ":" "<" "=" ">" "?" "A" "B" "C" "D"

"E" "F" "G" "H" "I" "J" "K" "L" "M" "N" "O" "P" "Q" "R" "S" "T" "U" "V" "W" "X" "Y" "Z" "[" "]" "_" "a" "b" "c" "d" "e" "f" "g" "h" "i" "j" "k" "l" "m" "n" "o" "p" "q" "r" "s" "t" "u" "v" "w" "x" "y" "z"

*Commands to get the display characters as text data (ASCII)

This command can get certain text data (ASCII) from the RX-Vx800 to be used by a host device as follows.

- Tuner frequency characters : " 1 07.9 "(MHz)

- Master volume value characters : " –40.0dB" / "  MUTE"

- Input name : " MY PC " (Even renamed by "SET MENU:INPUT RENAME")

- Zone2 input name : " PS 3 " (Even renamed by "SET MENU:INPUT RENAME")

The response protocol for the text request commands are as follows.

Text Char.

RX-Vx800	Host

| RCMD0,1 | COMMAND | 0 – 9,A - F | 0...0xFF |
| --- | --- | --- | --- |
| DDAT 0 - 7 | DATA | 0 – 9,A - Z SP | ASCII char. Space char. |

Report Command

|  | DC1 | RCMD0 | RCMD1 | DDAT 0 | DDAT 1 | DDAT 2 | DDAT 3 | DDAT 4 | DDAT 5 | DDAT 6 | DDAT 7 | ETX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tuner Frequency | DC1 | 0 | 0 | SP | SP | x | x | x | x | x | x | ETX |
| Main Volume Value | DC1 | 0 | 1 | SP | x | x | x | x | x | x | x | ETX |
| Zone2 Volume Value | DC1 | 0 | 2 | SP | x | x | x | x | x | x | x | ETX |
| Input Name | DC1 | 0 | 3 | x | x | x | x | x | x | x | x | ETX |
| Zone2 Input Name | DC1 | 0 | 4 | x | x | x | x | x | x | x | x | ETX |
| Zone3 Volume Value | DC1 | 0 | 5 | SP | x | x | x | x | x | x | x | ETX |
| Zone3 Input Name | DC1 | 0 | 6 | x | x | x | x | x | x | x | x | ETX |

13

RX-Vx800 RS-232C Protocol

3.2  Operation Commands ( SW = '0',  CMDT0 = '7' )

4 Byte Remote Control

STX	SW ( '0'  CMDT0 CMDT1

( '7' )

CMDT2 CMDT3	ETX

| SW |  | CMDT_ |  |  | Function | Setting | Report Command |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  | Type | RCMD1,2 |  |
| 0 | 7 | A | 1 | A | Main Volume | Up | 0 | 26 |  |
| 0 | 7 | A | 1 | B |  | Down |  |  |  |
| 0 | 7 | E | A | 2 | Audio Mute | On (Full) | 0 | 23 |  |
| 0 | 7 | E | D | F |  | On (-20dB) |  |  |  |
| 0 | 7 | E | A | 3 |  | Off |  |  |  |
| 0 | 7 | A | 1 | 4 | Input | PHONO | 0 | 21 |  |
| 0 | 7 | A | 1 | 5 |  | CD |  |  |  |
| 0 | 7 | A | 1 | 6 |  | TUNER |  |  |  |
| 0 | 7 | A | 1 | 9 |  | CD-R |  |  |  |
| 0 | 7 | A | 1 | 8 |  | MD/TAPE |  |  |  |
| 0 | 7 | A | C | 8 |  | BD/HD DVD |  |  |  |
| 0 | 7 | A | C | 1 |  | DVD |  |  |  |
| 0 | 7 | A | 5 | 4 |  | DTV /CBL |  |  |  |
| 0 | 7 | A | C | 0 |  | CBL/SAT |  |  | Deletion |
| 0 | 7 | A | 0 | F |  | VCR |  |  |  |
| 0 | 7 | A | 1 | 3 |  | DVR |  |  |  |
| 0 | 7 | A | 5 | 5 |  | V-AUX/DOCK |  |  |  |
| 0 | 7 | A | 8 | 7 |  | Multi CH |  |  |  |
| 0 | 7 | A | B | 4 |  | XM |  |  | Only USA & Canada |
| 0 | 7 | E | A | 4 | Multi CH Input | On | 0 | 21 |  |
| 0 | 7 | E | A | 5 |  | Off |  |  |  |
| 0 | 7 | E | 8 | 0 | Pure Direct | On | 0 | 8C |  |
| 0 | 7 | E | 8 | 2 |  | Off |  |  |  |
| 0 | 7 | E | A | 6 | Audio Select | Auto | 0 | 22 |  |
| 0 | 7 | E | A | 9 |  | COAX/OPT |  |  |  |
| 0 | 7 | E | A | A |  | Analog |  |  |  |
| 0 | 7 | E | D | A |  | HDMI |  |  |  |
| 0 | 7 | E | D | B | Decoder Mode | AUTO | 0 | 5E |  |
| 0 | 7 | E | A | 8 |  | DTS |  |  |  |
| 0 | 7 | E | 3 | B |  | AAC |  |  | Only Japanese Model |
| 0 | 7 | A | D | A | Zone2 Volume | Up | 0 | 27 |  |
| 0 | 7 | A | D | B |  | Down |  |  |  |
| 0 | 7 | E | A | 0 | Zone2 Mute | On | 0 | 25 |  |
| 0 | 7 | E | A | 1 |  | Off |  |  |  |
| 0 | 7 | A | D | 0 | Zone2 Input | PHONO | 0 | 24 |  |
| 0 | 7 | A | D | 1 |  | CD |  |  |  |
| 0 | 7 | A | D | 2 |  | TUNER |  |  |  |
| 0 | 7 | A | D | 4 |  | CD-R |  |  |  |
| 0 | 7 | A | D | 3 |  | MD/TAPE |  |  |  |
| 0 | 7 | A | C | E |  | BD/HD DVD |  |  |  |
| 0 | 7 | A | C | D |  | DVD |  |  |  |
| 0 | 7 | A | D | 9 |  | DTV /CBL |  |  |  |
| 0 | 7 | A | C | C |  | CBL/SAT |  |  | Deletion |
| 0 | 7 | A | D | 6 |  | VCR |  |  |  |
| 0 | 7 | A | D | 7 |  | DVR |  |  |  |
| 0 | 7 | A | D | 8 |  | V-AUX/DOCK |  |  |  |
| 0 | 7 | A | B | 8 |  | XM |  |  | Only USA & Canada |
| 0 | 7 | A | 1 | D | All Zone Power | On | 0 | 20 |  |
| 0 | 7 | A | 1 | E |  | Standby |  |  |  |
| 0 | 7 | E | 7 | E | Main Zone Power | On | 0 | 20 |  |
| 0 | 7 | E | 7 | F |  | Standby |  |  |  |
| 0 | 7 | E | B | A | Zone2 Power | On | 0 | 20 |  |
| 0 | 7 | E | B | B |  | Standby |  |  |  |
| 0 | 7 | A | E | D | Zone3 Power | On | 0 | 20 |  |
| 0 | 7 | A | E | E |  | STANDBY |  |  |  |
| 0 | 7 | E | 2 | 6 | Zone3 Mute | On | 0 | A1 |  |
| 0 | 7 | E | 6 | 6 |  | Off |  |  |  |
| 0 | 7 | A | F | D | Zone3 Volume | Up | 0 | A2 |  |
| 0 | 7 | A | F | E |  | Down |  |  |  |

14

RX-Vx800 RS-232C Protocol

| SW |  | CMDT_ |  |  | Function | Setting | Report Command |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  | Type | RCMD1,2 |  |
| 0 | 7 | A | F | 1 | Zone3 Input | PHONO | 0 | A0 |  |
| 0 | 7 | A | F | 2 |  | CD |  |  |  |
| 0 | 7 | A | F | 3 |  | TUNER |  |  |  |
| 0 | 7 | A | F | 5 |  | CD-R |  |  |  |
| 0 | 7 | A | F | 4 |  | MD/TAPE |  |  |  |
| 0 | 7 | A | F | B |  | BD/HD DVD |  |  |  |
| 0 | 7 | A | F | C |  | DVD |  |  |  |
| 0 | 7 | A | F | 6 |  | DTV /CBL |  |  |  |
| 0 | 7 | A | F | 7 |  | CBL/SAT |  |  | Deletion |
| 0 | 7 | A | F | 9 |  | VCR |  |  |  |
| 0 | 7 | A | F | A |  | DVR |  |  |  |
| 0 | 7 | A | F | 0 |  | V-AUX/DOCK |  |  |  |
| 0 | 7 | A | B | 9 |  | XM |  |  | Only USA & Canada |
| 0 | 7 | E | B | 0 | Short Message | Off | 0 | 2B |  |
| 0 | 7 | E | B | 1 |  | On |  |  |  |
| 0 | 7 | E | B | 2 |  | Full |  |  | Only RX-V1800 |
| 0 | 7 | E | B | 3 | Sleep Timer | Off | 0 | 2C |  |
| 0 | 7 | E | B | 4 |  | 120 |  |  |  |
| 0 | 7 | E | B | 5 |  | 90 |  |  |  |
| 0 | 7 | E | B | 6 |  | 60 |  |  |  |
| 0 | 7 | E | B | 7 |  | 30 |  |  |  |
| 0 | 7 | E | B | 8 | EXTD SUR. | EX/ES | 0 0 0 0 0 0 | 2D |  |
| 0 | 7 | E | B | 9 |  | Off |  |  |  |
| 0 | 7 | E | 7 | C |  | Auto |  |  |  |
| 0 | 7 | E | D | C |  | EX |  |  |  |
| 0 | 7 | E | D | D |  | PLIIx Movie |  |  |  |
| 0 | 7 | E | D | E |  | PLIIx Music |  |  |  |
| 0 | 7 | E | 9 | C | Night Listening Mode | Off | 0 | 8B | Deletion |
| 0 | 7 | E | 9 | B |  | Cinema |  |  |  |
| 0 | 7 | E | C | F |  | Music |  |  |  |
| 0 | 7 | E | 2 | 7 | EFFECT | ON | 0 | 28 |  |
| 0 | 7 | E | E | 0 | STRAIGHT |  | 0 | 28 |  |
| 0 | 7 | E | D | 8 | DSP / Surround | Straight Enhancer | 0 | 28 |  |
| 0 | 7 | E | D | 9 |  | Off |  |  | Deletion |
| 0 | 7 | E | E | 1 | DSP / Surround | Munich | 0 | 28 |  |
| 0 | 7 | E | E | 5 |  | Vienna |  |  |  |
| 0 | 7 | E | E | 6 |  | Amsterdam |  |  |  |
| 0 | 7 | E | E | 8 |  | Freiburg |  |  |  |
| 0 | 7 | E | A | F |  | Chamber |  |  |  |
| 0 | 7 | E | E | B |  | Village Vanguard |  |  |  |
| 0 | 7 | E | E | E |  | Warehouse Loft |  |  |  |
| 0 | 7 | E | C | D |  | Cellar Club |  |  |  |
| 0 | 7 | E | E | C |  | The Bottom Line |  |  |  |
| 0 | 7 | E | E | D |  | The Roxy Theatre |  |  |  |
| 0 | 7 | E | F | 0 |  | Disco |  |  |  |
| 0 | 7 | E | F | 2 |  | Game |  |  |  |
| 0 | 7 | E | F | F |  | 7ch Stereo |  |  |  |
| 0 | 7 | E | C | 0 |  | 2ch Stereo |  |  |  |
| 0 | 7 | E | F | 8 |  | Sports |  |  |  |
| 0 | 7 | E | F | 2 |  | Action Game |  |  |  |
| 0 | 7 | E | C | E |  | Roleplaying Game |  |  |  |
| 0 | 7 | E | F | 3 |  | Music Video |  |  |  |
| 0 | 7 | E | F | 5 |  | Recital/Opera |  |  |  |
| 0 | 7 | E | F | E |  | Standard |  |  |  |
| 0 | 7 | E | F | 9 |  | Spectacle |  |  |  |
| 0 | 7 | E | F | A |  | Sci-Fi |  |  |  |
| 0 | 7 | E | F | B |  | Adventure |  |  |  |
| 0 | 7 | E | F | C |  | Drama |  |  |  |
| 0 | 7 | E | F | 7 |  | Mono Movie |  |  |  |
| 0 | 7 | E | F | D |  | Surround Decode |  |  |  |
| 0 | 7 | E | C | 2 |  | THX Cinema |  |  |  |
| 0 | 7 | E | C | 3 |  | THX Music |  |  |  |
| 0 | 7 | E | C | 8 |  | THX Game |  |  |  |
| 0 | 7 | A | E | 0 | Tuner Preset Page | A | 0 | 29 |  |
| 0 | 7 | A | E | 1 |  | B |  |  |  |
| 0 | 7 | A | E | 2 |  | C |  |  |  |
| 0 | 7 | A | E | 3 |  | D |  |  |  |
| 0 | 7 | A | E | 4 |  | E |  |  |  |

15

RX-Vx800 RS-232C Protocol

| SW |  | CMDT_ |  |  | Function | Setting | Report Command |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  | Type | RCMD1,2 |  |
| 0 | 7 | A | E | 5 | Tuner Preset Number | 1 | 0 | 2A |  |
| 0 | 7 | A | E | 6 |  | 2 |  |  |  |
| 0 | 7 | A | E | 7 |  | 3 |  |  |  |
| 0 | 7 | A | E | 8 |  | 4 |  |  |  |
| 0 | 7 | A | E | 9 |  | 5 |  |  |  |
| 0 | 7 | A | E | A |  | 6 |  |  |  |
| 0 | 7 | A | E | B |  | 7 |  |  |  |
| 0 | 7 | A | E | C |  | 8 |  |  |  |
| 0 | 7 | E | B | C | Tuner Band | FM | 0 | 35 |  |
| 0 | 7 | E | B | D |  | AM |  |  |  |
| 0 | 7 | E | B | E | Tuner Auto Tuning | Up | 0 | 15 |  |
| 0 | 7 | E | B | F |  | Down |  |  |  |
| 0 | 7 | E | A | B | Speaker Relay A | On | 0 | 2E |  |
| 0 | 7 | E | A | C |  | Off |  |  |  |
| 0 | 7 | E | A | D | Speaker Relay B | On | 0 | 2F |  |
| 0 | 7 | E | A | E |  | Off |  |  |  |
| 0 | 7 | E | 2 | B | System Memory Save | 1 | 0 | 31 |  |
| 0 | 7 | E | 2 | C |  | 2 |  |  |  |
| 0 | 7 | E | 2 | D |  | 3 |  |  |  |
| 0 | 7 | E | 2 | E |  | 4 |  |  |  |
| 0 | 7 | E | 2 | F |  | 5 |  |  |  |
| 0 | 7 | E | 3 | 0 |  | 6 |  |  |  |
| 0 | 7 | E | 3 | 5 | System Memory Load | 1 | 0 | 30 |  |
| 0 | 7 | E | 3 | 6 |  | 2 |  |  |  |
| 0 | 7 | E | 3 | 7 |  | 3 |  |  |  |
| 0 | 7 | E | 3 | 8 |  | 4 |  |  |  |
| 0 | 7 | E | 3 | 9 |  | 5 |  |  |  |
| 0 | 7 | E | 3 | A |  | 6 |  |  |  |
| 0 | 7 | E | 6 | B | Main Volume Memory Save | 1 | 0 | 33 |  |
| 0 | 7 | E | 6 | C |  | 2 |  |  |  |
| 0 | 7 | E | 6 | D |  | 3 |  |  |  |
| 0 | 7 | E | 6 | E |  | 4 |  |  |  |
| 0 | 7 | E | 6 | F |  | 5 |  |  |  |
| 0 | 7 | E | 7 | 0 |  | 6 |  |  |  |
| 0 | 7 | E | 7 | 5 | Main Volume Memory Load | 1 | 0 | 32 |  |
| 0 | 7 | E | 7 | 6 |  | 2 |  |  |  |
| 0 | 7 | E | 7 | 7 |  | 3 |  |  |  |
| 0 | 7 | E | 7 | 8 |  | 4 |  |  |  |
| 0 | 7 | E | 7 | 9 |  | 5 |  |  |  |
| 0 | 7 | E | 7 | A |  | 6 |  |  |  |
| 0 | 7 | E | 8 | 7 | Zone2 Volume Memory Save | 1 | 0 | 38 |  |
| 0 | 7 | E | 8 | 8 |  | 2 |  |  |  |
| 0 | 7 | E | 8 | 9 |  | 3 |  |  |  |
| 0 | 7 | E | 8 | A |  | 4 |  |  |  |
| 0 | 7 | E | 8 | B |  | 5 |  |  |  |
| 0 | 7 | E | 8 | C |  | 6 |  |  |  |
| 0 | 7 | E | 8 | D | Zone2 Volume Memory Load | 1 | 0 | 37 |  |
| 0 | 7 | E | 8 | E |  | 2 |  |  |  |
| 0 | 7 | E | 8 | F |  | 3 |  |  |  |
| 0 | 7 | E | 9 | 0 |  | 4 |  |  |  |
| 0 | 7 | E | 9 | 1 |  | 5 |  |  |  |
| 0 | 7 | E | 9 | 2 |  | 6 |  |  |  |
| 0 | 7 | E | 2 | 0 | Zone3 Volume Memory Save | 1 | 0 | A4 |  |
| 0 | 7 | E | 2 | 1 |  | 2 |  |  |  |
| 0 | 7 | E | 2 | 2 |  | 3 |  |  |  |
| 0 | 7 | E | 2 | 3 |  | 4 |  |  |  |
| 0 | 7 | E | 2 | 4 |  | 5 |  |  |  |
| 0 | 7 | E | 2 | 5 |  | 6 |  |  |  |
| 0 | 7 | E | 6 | 0 | Zone3 Volume Memory Load | 1 | 0 | A3 |  |
| 0 | 7 | E | 6 | 1 |  | 2 |  |  |  |
| 0 | 7 | E | 6 | 2 |  | 3 |  |  |  |
| 0 | 7 | E | 6 | 3 |  | 4 |  |  |  |
| 0 | 7 | E | 6 | 4 |  | 5 |  |  |  |
| 0 | 7 | E | 6 | 5 |  | 6 |  |  |  |
| 0 | 7 | E | D | 4 | Zone2 Balance | Left | 0 | AD |  |
| 0 | 7 | E | D | 5 |  | Right |  |  |  |

16

RX-Vx800 RS-232C Protocol

| SW |  | CMDT_ |  |  | Function | Setting | Report Command |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  | Type | RCMD1,2 |  |
| 0 | 7 | E | D | 6 | Zone3 Balance | Left | 0 | AE |  |
| 0 | 7 | E | D | 7 |  | Right |  |  |  |
| 0 | 7 | E | 3 | 2 | Trigger1 Control | Main | 0 | 3A |  |
| 0 | 7 | E | 3 | 3 |  | Zone2 |  |  |  |
| 0 | 7 | E | 3 | 1 |  | Zone3 |  |  |  |
| 0 | 7 | E | 3 | 4 |  | All (Zone1-3) |  |  |  |
| 0 | 7 | E | 7 | 3 | Trigger1 Main | High | 0 | 36 |  |
| 0 | 7 | E | 7 | 4 |  | Low |  |  |  |
| 0 | 7 | E | 7 | 1 | Trigger1 Zone2 | High | 0 | 36 |  |
| 0 | 7 | E | 7 | 2 |  | Low |  |  |  |
| 0 | 7 | E | 8 | 3 | Trigger1 Zone3 | High | 0 | 36 |  |
| 0 | 7 | E | 8 | 4 |  | Low |  |  |  |
| 0 | 7 | E | 9 | 3 | Dual Mono Mode | Main | 0 | 39 | Only Japanese Model |
| 0 | 7 | E | 9 | 4 |  | Sub |  |  |  |
| 0 | 7 | E | 9 | 5 |  | All |  |  |  |
| 0 | 7 | E | 9 | 6 | Trigger2 Control | Main | 0 | 3B |  |
| 0 | 7 | E | 9 | 7 |  | Zone2 |  |  |  |
| 0 | 7 | E | 9 | F |  | Zone3 |  |  |  |
| 0 | 7 | E | 9 | 8 |  | All (Zone1-3) |  |  |  |
| 0 | 7 | E | 3 | E | Trigger2 Main | High | 0 | 3C |  |
| 0 | 7 | E | 3 | F |  | Low |  |  |  |
| 0 | 7 | E | 3 | C | Trigger2 Zone2 | High | 0 | 3C |  |
| 0 | 7 | E | 3 | D |  | Low |  |  |  |
| 0 | 7 | E | 8 | 5 | Trigger2 Zone3 | High | 0 | 3C |  |
| 0 | 7 | E | 8 | 6 |  | Low |  |  |  |
| 0 | 7 | E | 2 | 8 | Speaker B SET | Main | 0 | 3D |  |
| 0 | 7 | E | 2 | 9 |  | Zone B |  |  |  |
| 0 | 7 | E | 9 | 9 | ZONE2 Amp | INT: Presence | 0 | 3E |  |
| 0 | 7 | E | 9 | A |  | EXT |  |  |  |
| 0 | 7 | E | 6 | 7 | 2ch Decoder | PLIIx Movie | 0 | 6E |  |
| 0 | 7 | E | 6 | 8 |  | PLIIx Music |  |  |  |
| 0 | 7 | E | 6 | 9 |  | Neo:6 Cinema |  |  |  |
| 0 | 7 | E | 6 | A |  | Neo:6 Music |  |  |  |
| 0 | 7 | E | C | 7 |  | PLIIx Game |  |  |  |
| 0 | 7 | E | C | 9 |  | Pro Logic |  |  |  |
| 0 | 7 | E | C | A |  | CSII Cinema |  |  | Only Japanese Model |
| 0 | 7 | E | C | B |  | CSII Music |  |  | Only Japanese Model |
| 0 | 7 | E | C | C |  | Neural Sur. |  |  |  |
| 0 | 7 | A | 6 | 0 | XM CH Number XM CH/Preset Number XM CH/Preset Number XM CH/Preset Number XM CH/Preset Number XM CH/Preset Number XM CH/Preset Number XM CH/Preset Number XM CH/Preset Number XM CH Number | 0 | 0 |  |  |
| 0 | 7 | A | 6 | 1 |  | 1 | 0 | 91* | *Preset Number |
| 0 | 7 | A | 6 | 2 |  | 2 | 0 | 91* |  |
| 0 | 7 | A | 6 | 3 |  | 3 | 0 | 91* |  |
| 0 | 7 | A | 6 | 4 |  | 4 | 0 | 91* |  |
| 0 | 7 | A | 6 | 5 |  | 5 | 0 | 91* |  |
| 0 | 7 | A | 6 | 6 |  | 6 | 0 | 91* |  |
| 0 | 7 | A | 6 | 7 |  | 7 | 0 | 91* |  |
| 0 | 7 | A | 6 | 8 |  | 8 | 0 | 91* |  |
| 0 | 7 | A | 6 | 9 |  | 9 | 0 |  |  |
| 0 | 7 | A | 6 | A | XM CH/Preset Number | Up | 0 | 91 / 94 | 91:Preset Number 94:Channel Number |
| 0 | 7 | A | 6 | B |  | Down |  |  |  |
| 0 | 7 | A | 6 | C | XM Category / Preset Page | Up | 0 | 90 | 90:Preset Page |
| 0 | 7 | A | 6 | E |  | Down |  |  |  |
| 0 | 7 | A | 6 | F | XM Hold/Release Display | Toggle | 0 | 95 |  |
| 0 | 7 | A | 7 | 0 | XM History Next | Toggle |  |  |  |
| 0 | 7 | A | 7 | 1 | XM Preset Memory |  |  |  |  |
| 0 | 7 | A | B | 5 | XM Search Mode | All CH | 0 | 92 |  |
| 0 | 7 | A | B | 6 |  | Category |  |  |  |
| 0 | 7 | A | B | 7 |  | Preset |  |  |  |
| 0 | 7 | A | B | A | XM Preset Page | A | 0 | 90 |  |
| 0 | 7 | A | B | B |  | B |  |  |  |
| 0 | 7 | A | B | C |  | C |  |  |  |
| 0 | 7 | A | B | D |  | D |  |  |  |
| 0 | 7 | A | B | E |  | E |  |  |  |
| 0 | 7 | A | B | F | XM ENT. |  |  |  | Decide CH Number |

17

RX-Vx800 RS-232C Protocol

| SW |  | CMDT_ |  |  | Function | Setting | Report Command |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  | Type | RCMD1,2 |  |
| 0 | 7 | A | 7 | 3 | Zone2 Tone Control | Bass + | 0 | 4B |  |
| 0 | 7 | A | 7 | 4 |  | Bass - |  |  |  |
| 0 | 7 | A | 7 | 5 |  | Treble + | 0 | 4C |  |
| 0 | 7 | A | 7 | 6 |  | Treble - |  |  |  |
| 0 | 7 | A | 7 | 7 | Zone3 Tone Control | Bass + | 0 | 4D |  |
| 0 | 7 | A | 7 | 8 |  | Bass - |  |  |  |
| 0 | 7 | A | 7 | 9 |  | Treble + | 0 | 4E |  |
| 0 | 7 | A | 7 | A |  | Treble - |  |  |  |
| 0 | 7 | A | A | 0 | GUI Operation | Top Menu |  | No Report |  |
| 0 | 7 | A | D | E |  | Enter |  |  |  |
| 0 | 7 | A | A | 1 |  | Exit |  |  |  |
| 0 | 7 | A | 9 | D |  | Cursor Up |  |  |  |
| 0 | 7 | A | 9 | C |  | Cursor Down |  |  |  |
| 0 | 7 | A | 9 | E |  | Cursor Right |  |  |  |
| 0 | 7 | A | 9 | F |  | Cursor Left |  |  |  |
| 0 | 7 | E | D | 0 | Main Source Display | On | 0 | No Report |  |
| 0 | 7 | E | D | 1 |  | Off |  |  |  |
| 0 | 7 | E | D | 2 | Zone2 Source Display | On | 0 | No Report | Only RX-V3800 |
| 0 | 7 | E | D | 3 |  | Off |  |  |  |

18

RX-Vx800 RS-232C Protocol

3.3  Extended Operation Command ( SW = '0', CMDT0 = 'F' )

9 Byte Remote Control

STX	SW ( '0'  CMDT0 CMDT1 CMDT2 CMDT3 CMDT4 CMDT5 CMDT6 CMDT7 CMDT8	ETX

( 'F' )

| SW |  |  |  | CMDT_ |  |  |  |  |  | Function | Setting | Report Command |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 4 |  | 5 | 6 7 |  | 8 |  |  | Type | RCMD1,2 |  |
| 0 | F | 7 | F | 0 | 1 | 0 | F | F | 0 | iPod OSD (GUI) Operation | Menu |  | No Report |  |
| 0 | F | 7 | F | 0 | 1 | 1 | 1 | E | E |  | Enter |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | 5 | E | A |  | Display |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 0 | E | F | 1 |  | Cursor Up |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | 4 | E | B |  | Cursor Down |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | 2 | E | D |  | Cursor Right |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | 0 | E | F |  | Cursor Left |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | E | E | 1 | iPod Operation | Play |  | No Report |  |
| 0 | F | 7 | F | 0 | 1 | 1 | D | E | 2 |  | Stop |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | A | E | 5 |  | Pause |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | C | E | 3 |  | Skip + |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 1 | B | E | 4 |  | SKip - |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | F | C | 0 | Input | NET/USB | 0 | 21 | Only RX-V3800 |
| 0 | F | 7 | F | 0 | 1 | 4 | 0 | B | F | Zone2 Input | NET/USB | 0 | 24 | Only RX-V3800 |
| 0 | F | 7 | F | 0 | 1 | 4 | 1 | B | E | Zone3 Input | NET/USB | 0 | A0 | Only RX-V3800 |
| 0 | F | 7 | F | 0 | 1 | 3 | 6 | C | 9 | NET/USB Source Select | PC/MCX | 0 | 9B | Only RX-V3800 |
| 0 | F | 7 | F | 0 | 1 | 3 | 7 | C | 8 |  | NET Radio |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | 8 | C | 7 |  | USB |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 2 | F | D | 0 | NET/USB OSD (GUI) Operation | Menu |  | No Report | Only RX-V3800 |
| 0 | F | 7 | F | 0 | 1 | 3 | 1 | C | E |  | Enter |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | 5 | C | A |  | Display |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 2 | E | D | 1 |  | Cursor Up |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | 4 | C | B |  | Cursor Down |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | 2 | C | D |  | Cursor Right |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | 0 | C | F |  | Cursor Left |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | E | C | 1 | NET/USB Operation | Play |  | No Report | Only RX-V3800 |
| 0 | F | 7 | F | 0 | 1 | 3 | D | C | 2 |  | Stop |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | C | C | 3 |  | Skip + |  |  |  |
| 0 | F | 7 | F | 0 | 1 | 3 | B | C | 4 |  | SKip - |  |  |  |
| 0 | F | 7 | A | 8 | 5 | 0 | 0 | 7 | F | System Memory Operation | Play | 0 | 30 (Load) 31 (Save) | Only RX-V3800 |
| 0 | F | 7 | A | 8 | 5 | 0 | 3 | 7 | C |  | Stop |  |  |  |
| 0 | F | 7 | A | 8 | 5 | 0 | 6 | 7 | 9 |  | Skip + |  |  |  |
| 0 | F | 7 | A | 8 | 5 | 0 | 9 | 7 | 6 |  | SKip - |  |  |  |
| 0 | F | 7 | E | 8 | 1 | 1 | 4 | 6 | B | DSP / Surround | 7ch Enhancer | 0 | 28 |  |

19

RX-Vx800 RS-232C Protocol

4. Reset Command

The Reset Command recalls factory preset data. Once the factory presets are recalled, all user controllable setting / parameter data will be deleted and replaced with original factory settings.

Do not use this command unless you have been experiencing problems with the system or if you just want to clean up the system.

RX-Vx800	Reset	Host

After the system is reset, request the Configuration Command using the Ready Command (see section 2) in order to get accurate feedback of RX-Vx800 status to your touch panel system.

Ready	DC3	DEL	DEL	DEL	ETX	*recall factory preset of all data

20

RX-Vx800 RS-232C Protocol

5. Report Command

The RX-Vx800 will send a Report Command in response to Control Commands from the host controller. From the Report Command you can receive the current status of the RX-Vx800.

Control Command

RX-Vx800	Host

Report Command

There are three types of Report Commands classified by their information type.

- System Status Report

- Playback Status Report

- Operation Report

RX-Vx800 reports a System Status Report when the system status has changed.

RX-Vx800 reports a Playback Status Report when the internal playback status has changed.

When the RX-Vx800 is controlled by remote controller, front panel, RS-232C or system controller, the RX-Vx800 sends an Operation Report, which includes the latest setting status of the controlled function.

*The RX-Vx800 reports a System State Report with system guard to inform its power status (power off) if it receives a control command while

it is turned off.

*The guard status is included in the Report Command (GRD). If the control command the host sent was accepted, the guard status in the Report

Command is '0' (No Guard).

On the contrary the guard status will be 'System Guard' or Setting Guard' if the command was guarded for some reason.

(e.g. If you send a 'Speaker A ON' command while you are using a headphone, the guard status will be 'System Guard' because the speaker

controls are prohibited when a headphone is being used).

*If a status changes multiple times in a certain time, the RX-Vx800 reports only one report command.

Report Command

STX	TYP	GRD	RCMD0 RCMD1  RDAT0  RDAT1	ETX

Command	Data

| function name | function | data (ASCII) | range (HEX) |
| --- | --- | --- | --- |
| TYP | control type | 0 - 9 | 0 - 9 |
| GRD | guard status | 0 - 9 | 0 - 9 |
| RCMD0, 1 | command | 0 - 9, A - F | 0 - 0xFF |
| RDAT0, 1 | data | 0 - 9, A - F | 0 - 0xFF |

<Control type> This indicates for which type of control the report command is.

| TYP | control type |
| --- | --- |
| 0 | controlled by RS-232C |
| 1 | ) controlled by remote controller (IR |
| 2 | controlled by keys in the unit |
| 3 | controlled by system |
| 4 | controlled by encoder |

<Guard status> This indicates guard status against all control commands

| GRD | Guard status* |
| --- | --- |
| 0 | no guard |
| 1 | system guard |
| 2 | setting guard |

21

RX-Vx800 RS-232C Protocol

5.1 System Status Reports

Report Item	Status

STX	TYP	GRD	RCMD0	RCMD1	RDAT0	RDAT1	ETX

| RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- |
| 00 | system | 00 01 02 | OK Busy Standby |

*RX-Vx800 sends this report when the system is reset or the power turns off.

It can be used for observation of the system revival.

| RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- |
| 01 | warning | 00 01 02 03 | Over Current DC Detect Power Trouble Over Heat |

| RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- |
| 06 | XM Message | 00 01 02 03 04 05 | Check Antenna Updating No Signal Loading Off Air Unavailable |
| 07 | iPod Message | 00 01 02 03 04 05 | Loading Connect Error Unknown iPod iPod connected Disconnected Unable to play |
| 08 | NET/USB Message | 00 01 02 03 04 05 06 07 08 09 0A | Please wait Connected Disconnected Access error List updated Startup server Unable to play Connect error Bookmark ON Bookmark OFF Not found |

Ready to accept control commands

Start of the term prohibits sending commands

Report against commands which cannot be accepted when Power is on Standby

Report of abnormal states

(Only when it's possible to report)

22

RX-Vx800 RS-232C Protocol

5.2 Playback Status Reports

Report Item	Status

STX	TYP	GRD	RCMD0	RCMD1	RDAT0	RDAT1	ETX

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Format | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C FE FF | Analog PCM DSD Digital Dolby Digital DTS AAC DTS-HD DTS-HD MSTR DD Plus TrueHD WMA MP3 ??? --- | 14 | Bit Rate | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F 20 21 22 23 24 25 26 FF | 32kbps 40kbps 48kbps 56kbps 64kbps 72kbps 80kbps 96kbps 112kbps 128kbps 144kbps 160kbps 192kbps 224kbps 256kbps 288kbps 320kbps 384kbps 448kbps 512kbps 576kbps 640kbps 768kbps 960kbps 1024kbps 1152kbps 1280kbps 1344kbps 1408kbps 1411.2kbps 1472kbps 1536kbps 1920kbps 2048kbps 3072kbps 3840kbps Open Variable Loss less --- |
| 11 | Sampling | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 FE FF | Analog 8kHz 11.025kHz 12kHz 16kHz 22.05kHz 24kHz 32kHz 44.1kHz 48kHz 64kHz 88.2kHz 96kHz 128kHz 176.4kHz 192kHz 2.8MHz (DSD) ??? --- |  |  |  |  |
| 12 | Channel Front/Rear | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0F | 1+1 1/0 2/0 3/0 2/1 3/1 2/2 3/2 2/3 3/3 2/4 3/4 MLT --- |  |  |  |  |
|  |  |  |  | 15 | Dialog | 00 01 … 1E 1F FF | -31dB -30dB -1dB +0dB --- |
|  |  |  |  | 16 | Flag | xx |  |
|  |  |  |  |  |  | bit0 bit1 bit2 bit3 bit4 bit5 bit6 bit7 | DD Karaoke DD 6.1 dts ES Matrix 6.1 dts ES Discrete 6.1 dts 96kHz/24bit Pre Emphasis DPL Encoded Don't Care |
| 13 | Channel LFE | 00 01 FF | 0.1 0.2 --- |  |  |  |  |

23

RX-Vx800 RS-232C Protocol

5.3 Operation Reports

Report Item	Status

STX	TYP	GRD	RCMD0	RCMD1	RDAT0	RDAT1	ETX

| RCMD0, 1 | Report Item | RDAT0, 1 |  | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | Power | 00 01 02 03 04 05 06 07 | ALL (Main/Zone2/3) OFF ALL (Main/Zone2/3) ON Main On / Zone2 Off / Zone3 Off Main Off / Zone2 On / Zone3 On Main On / Zone2 On / Zone3 Off Main On / Zone2 Off / Zone3 On Main Off / Zone2 On / Zone3 Off Main Off / Zone2 Off / Zone3 On |  | 27 | Zone 2 Vol. | 00 27 … C7 … E8 | -oo (Infinite Attenuation) -80dB 0dB 16.5dB |
|  |  |  |  |  | 28 | Program | 00 01 02 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 14 15 16 17 18 19 1C 1E 1F 1D 20 21 24 25 28 29 2C 2D 30 31 32 33 34 35 36 37 3C | Munich Hall B Hall C Hall D Vienna Live Concert Hall in Amsterdam Tokyo Freiburg Royaumont Chamber Village Gate Village Vanguard The Bottom Line Cellar Club The Roxy Theater Warehouse Loft Arena Disco Party Game 7ch Stereo Music Video DJ Recital/Opera Action Game Roleplaying Game Pavilion Mono Movie Sports Spectacle Sci-Fi Adventure Drama Surround Decode Standard PLII Movie PLII Music Neo: 6 Movie Neo: 6 Music 2ch Stereo Stereo B 2ch Direct Stereo THX Cinema THX Music THX Game |
| 21 | Input | 00 01 02 03 04 05 06 | PHONO CD TUNER CD-R MD/TAPE DVD DTV/CBL |  |  |  |  |  |
|  |  | 07 | CBL/SAT |  |  |  |  |  |
|  |  | 08 09 0A 0B 0C 0D 0E 10 0F 0/1, x | SAT VCR DVR VCR3/DVR V-AUX/DOCK NET/USB XM Multi CH BD/HD DVD Multi CH input OFF/ON |  |  |  |  |  |
| 22 | Audio Select | x, 0 x, 3 x, 4 x, 5 x, 8 | AUTO COAX / OPT ANALOG ANALOG ONLY HDMI |  |  |  |  |  |
|  | Decoder Mode | 0, x 1, x 2, x | AUTO DTS AAC |  |  |  |  |  |
| 23 | Audio Mute | 00 01 | Off On |  |  |  |  |  |
| 24 | Zone2 Input | 00 01 02 03 04 05 06 07 08 09 0A 0C 0D 0E 0F | PHONO CD TUNER CD-R MD/TAPE DVD DTV CBL/SAT SAT VCR1 DVR/VCR2 V-AUX/DOCK NET/USB XM BD/HD DVD |  |  |  |  |  |
| 25 | Zone2 Mute | 00 01 | Off On |  |  |  | 40 | Enhancer 2ch Low |
|  |  |  |  |  |  |  | 41 | Enhancer 2ch High |
| 26 | Main Volume | 00 27 … C7 … E8 | -oo (Infinite Attenuation) -80dB 0dB 16.5dB |  |  |  | 42 | Enhancer 7ch Low |
|  |  |  |  |  |  |  | 43 | Enhancer 7ch High |
|  |  |  |  |  |  |  | 42 43 80 | Enhancer 7ch Low Enhancer 7ch High STRAIGHT |

24

RX-Vx800 RS-232C Protocol

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 29 | Tuner Page | 00 01 02 03 04 | A B C D E | 2C | Sleep | 00 01 02 03 04 | 120 90 60 30 Off |
| 2A | Preset No. | 00 01 02 03 04 05 06 07 | 1 2 3 4 5 6 7 8 | 2D | EXTD SUR | 00 01 02 03 04 05 06 | Off EX/ES Discrete On Auto EX PLIIx Movie PLIIx Music |
|  |  |  |  | 2E | SP Relay A | 00 01 | Off On |
| 2B | Short Message | 00 01 02 | Full On Off |  |  |  |  |
|  |  |  |  | 2F | SP Relay B | 00 01 | Off On |

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | System Memory | 01 02 03 04 05 06 | Load 1 2 3 4 5 6 | 37 | Zone 2 Vol. Memory | 01 02 03 04 05 06 | Load 1 2 3 4 5 6 |
| 31 | System Memory | 01 02 03 04 05 06 | Save 1 2 3 4 5 6 | 38 | Zone 2 Vol. Memory | 01 02 03 04 05 06 | Save 1 2 3 4 5 6 |
| 32 | Volume Memory | 01 02 03 04 05 06 | Load 1 2 3 4 5 6 | 39 | Dual Mono | 00 01 02 | Main Sub All |
|  |  |  |  | 3A | Trigger1 Control | 00 01 02 03 | All (Zone1-3) Main Zone 2 Zone 3 |
| 33 | Volume Memory | 01 02 03 04 05 06 | Save 1 2 3 4 5 6 |  |  |  |  |
|  |  |  |  | 3B | Trigger2 Control | 00 01 02 03 | All (Zone1-3) Main Zone2 Zone3 |
|  |  |  |  | 3C | Trigger2 Output | 00 01 | Low (Due to the delay) High (Due to the delay) |
| 34 | Headphone | 00 01 | Off On |  |  |  |  |
|  |  |  |  | 3D | SP B SET | 00 01 | Main Zone B |
| 35 | Tuner Band | 00 01 | FM AM |  |  |  |  |
|  |  |  |  | 3E | Zone 2 Amp | 00 01 02 03 | EXT INT: Speaker[1] INT: Speaker[2] INT: Both |
| 36 | Trigger1 Output | 00 01 | Low (Due to the delay) High (Due to the delay) |  |  |  |  |
|  |  |  |  | 3F | Zone 3 Amp | 00 01 02 03 | EXT INT: Speaker[1] INT: Speaker[2] INT: Both |

25

RX-Vx800 RS-232C Protocol

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | Level Front R | 14 15 … 3C | -10dB -9.5dB +10dB | 47 | Level Presence R | 14 15 … 3C | -10dB -9.5dB +10dB |
| 41 | Level Front L | 14 15 … 3C | -10dB -9.5dB +10dB | 48 | Level Presence L | 14 15 … 3C | -10dB -9.5dB +10dB |
| 42 | Level Center | 14 15 … 3C | -10dB -9.5dB +10dB | 49 | Level Subwoofer | 14 15 … 3C | -10dB -9.5dB +10dB |
| 43 | Level Surround R | 14 15 … 3C | -10dB -9.5dB +10dB | 4B | Zone2 Tone Control Bass | 00 01 … 14 | -10dB -9dB +10dB |
| 44 | Level Surround L | 14 15 … 3C | -10dB -9.5dB +10dB | 4C | Zone2 Tone Control Treble | 00 01 … 14 | -10dB -9dB +10dB |
| 45 | Level Surround Back R | 14 15 … 3C | -10dB -9.5dB +10dB | 4D | Zone3 Tone Control Bass | 00 01 … 14 | -10dB -9dB +10dB |
| 46 | Level Surround Back L | 14 15 … 3C | -10dB -9.5dB +10dB | 4E | Zone3 Tone Control Treble | 00 01 … 14 | -10dB -9dB +10dB |

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | Main L/R Balance | 00 ... 14 ... 28 | Lch Max Mid Rch Max | 5B | Initial Volume | 00 01 27 … C7 … E8 | Off -oo (Infinite Attenuation) -80dB 0dB 16.5dB |
| 51 | LFE Level SP | 00 01 ... 14 | -20dB -19dB 0dB |  |  |  |  |
|  |  |  |  | 5C | Extended Surround Set | 00 01 | Auto Off |
| 52 | LFE Level HP | 00 01 ... 14 | -20dB -19dB 0dB | 5D | Adaptive DSP Level | 00 01 | Auto Off |
|  |  |  |  | 5E | Decoder Mode | 0, x 1, x 2, x | Auto DTS AAC |
| 53 | Manual Audio Delay | 00 01 ... F0 | 0ms 1ms 240ms |  |  |  |  |
|  |  |  |  |  |  | x, 0 x, 1 x, 2 x, 3 x, 4 x, 5 x, 6 x, 7 x, 9 x, A x, C x, E | PHONO CD TUNER CD-R MD/TAPE DVD DTV CBL/SAT VCR1 DVR / VCR2 V-AUX XM |
| 56 | HDMI Auto Audio Delay | 00 01 ... F0 | 0ms 1ms 240ms |  |  |  |  |
| 57 | DSP 3D | 00 01 | OFF ON |  |  |  |  |
| 58 | Wall Paper | 00 0E 0F | YES Gray NONE |  |  |  |  |
| 59 | HDMI AUTO Lipsync | 00 01 | Auto Off | 5F | Decoder Mode Set | 00 01 | Auto Last |
| 5A | MAX Volume | 00 01 02 ... 08 09 0A | -30dB -25dB -20dB 10dB 15dB 16.5dB |  |  |  |  |

26

RX-Vx800 RS-232C Protocol

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | Audio Select Set | 00 01 | Auto Last | 68 | Memory Guard | 00 01 | Off On |
| 61 | Dimmer | 00 01 02 03 04 | -4 -3 -2 -1 0 | 69 | Video Conversion | 00 01 | Off On |
|  |  |  |  | 6B | Zone3 Volume Out | 00 01 | Variable Fixed |
|  |  |  |  | 6C | Zone OSD | 00 01 02 | Off Zone2 Zone2 & Zone3 |
| 62 | OSD Shift (RX-V1800) | 00 … 0A | -5 5 |  |  |  |  |
|  |  |  |  | 6D | Adaptive DRC | 00 01 | Auto Off |
|  | GUI Position (RX-V3800) | x, 0 … x, A | Horizontal -5 5 |  |  |  |  |
|  |  |  |  | 6E | 2ch Decoder | 00 01 02 03 04 05 06 07 09 | Pro Logic PLIIx Movie PLIIx Music PLIIx Game Neo:6 Cinema Neo:6 Music CSII Cinema CSII Music Neural Sur. |
|  |  | 0, x … A, x | Vertical -5 5 |  |  |  |  |
| 63 | Gray Back | 00 01 | Off Auto |  |  |  |  |
| 64 | Dynamic Range SP | 00 01 02 | Max. Std. Min. |  |  |  |  |
|  |  |  |  | 6F | Multi CH BGV | 00 01 05 06 | Off Last DVD DTV/CBL |
| 65 | Dynamic Range HP | 00 01 02 | Max. Std. Min. |  |  |  |  |
| 66 | Zone2 Volume Out | 00 01 | Variable Fixed |  |  | 07 | CBL/SAT |
|  |  |  |  |  |  | 09 0A 0C 0F | VCR DVR V-AUX BD/HD DVD |
| 67 | FL Scroll | 00 01 | Continue Once |  |  |  |  |

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 70 | Center SP | 00 01 02 | Large Small None | 7B | Multi CH Select | 00 02 03 04 05 06 | 6ch 8ch CD 8ch CD-R 8ch MD/TAPE 8ch DVD 8ch DTV/CBL |
| 71 | Front | 00 01 | Large Small |  |  |  |  |
| 72 | Surround SP | 00 01 02 | Large Small None |  |  |  |  |
|  |  |  |  |  |  | 07 | 8ch CBL/SAT |
|  |  |  |  |  |  | 09 0A 0C 0F | 8ch VCR 8ch DVR 8ch V-AUX 8ch BD/HD DVD |
| 73 | Surround Back | 00 01 02 03 04 | Large x2 Large x1 Small x2 Small x1 None |  |  |  |  |
|  |  |  |  | 7D | PR/SB Priority | 00 01 | Presence Surround Back |
| 74 | Presence | 00 01 | Yes None | 7E | Subwoofer Crossover | 00 01 02 03 04 05 06 07 08 | 40 Hz 60 Hz 80 Hz 90 Hz 100 Hz 110 Hz 120 Hz 160 Hz 200 Hz |
| 75 | LFE Bass Out | 00 01 02 | SWFR Main Both |  |  |  |  |
| 76 | Subwoofer Phase | 0, x 1, x | Normal Reverse |  |  |  |  |

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | Test | 00 01 | Off On | 8A | THX SB Dist | 00 01 02 | Under 1f (0.3m) 1 – 4ft (0.3 – 1.2m) Over 4ft (1.2m) |
| 85 | Component I/P | 00 01 | Off On |  |  |  |  |
|  |  |  |  | 8B | Night Mode | 00 | Off |
| 86 | HDMI I/P | 00 01 | Off On |  | Parameter | 10 11 | Cinema level Low Middle |
| 87 | HDMI Up-Scaling | 00 01 02 03 04 | Through 480p / 576p (NTSC / PAL) (*2) 1080i 720p 1080p |  |  | 12 | High |
|  |  |  |  |  |  | 20 | Music Level Low |
|  |  |  |  |  |  | 21 | Middle |
|  |  |  |  |  |  | 22 | High |
|  |  |  |  | 8C | Pure Direct | 00 01 | Off On |
| 88 | HDMI Aspect | 00 01 02 | Through 16:9 Normal Smart Zoom |  |  |  |  |
|  |  |  |  | 8F | HDMI Support Audio | 00 01 | RX-V3800 / RX-V1800 Other |

27

RX-Vx800 RS-232C Protocol

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | XM Preset Page | 00 01 02 03 04 | A B C D E | 96 | iPod Charge on Standby | 00 01 | Off Auto |
|  |  |  |  | 97 | iPod Repeat | 00 01 02 | Off One All |
| 91 | XM Preset Num | 00 01 02 03 04 05 06 07 | 1 2 3 4 5 6 7 8 | 98 | iPod Shuffle | 00 01 02 | Off Songs Albums |
|  |  |  |  | 99 | NET/USB Repeat | 00 01 02 | Off Single All |
|  |  |  |  | 9A | NET/USB Shuffle | 00 01 | Off On |
| 92 | XM Search Mode | 00 01 02 | All CH Category Preset | 9B | NET/USB Source Select | 00 01 02 | PC/MCX NET Radio USB |
| 93 | On Screen | 00 01 02 03 | Off 10sec 30sec Always | 9C | NET/USB Preset Recall | 00 01 02 … 08 | Parameter Error 1 2 8 |
| 94 | XM CH Number | 00 01 … FF | CH #0 CH #1 CH #255 |  |  |  |  |
|  |  |  |  | 9D | NET/USB Preset Memory | 00 01 02 … 08 | Parameter Error 1 2 8 |
| 95 | XM Display Hold / Release | 00 01 | Release Hold |  |  |  |  |

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A0 | Zone 3 Input | 00 01 02 03 04 05 06 07 09 0A 0C 0D 0E | PHONO CD TUNER CD-R MD/TAPE DVD DTV CBL/SAT VCR1 DVR/VCR2 V-AUX/DOCK NET/USB XM | A9 | Zone2 MAX Volume | 00 01 02 … 08 09 0A | -30dB -25dB -20dB 10dB 15dB 16.5dB |
|  |  |  |  | AA | Zone3 MAX Volume | 00 01 02 … 08 09 0A | -30dB -25dB -20dB 10dB 15dB 16.5dB |
| A1 | Zone 3 Mute | 00 01 | Off On |  |  |  |  |
|  |  |  |  | AB | Zone2 Initial Volume | 00 01 27 … C7 … E8 | Off -oo (Infinite Attenuation) -80dB 0dB 16.5dB |
| A2 | Zone 3 Volume | 00 27 … C7 … E8 | -oo (Infinite Attenuation) -80dB 0dB 16.5dB |  |  |  |  |
| A3 | Zone 3 Volume Memory | 01 02 03 04 05 06 | Load 1 2 3 4 5 6 | AC | Zone3 Initial Volume | 00 01 27 … C7 … E8 | Off -oo (Infinite Attenuation) -80dB 0dB 16.5dB |
| A4 | Zone 3 Volume Memory | 01 02 03 04 05 06 | Save 1 2 3 4 5 6 |  |  |  |  |
|  |  |  |  | AD | Zone2 Balance | 00 01 … 14 … 27 28 | L20 L19 Mid R19 R20 |
| A5 | Mute Type | 00 01 | Full -20dB |  |  |  |  |
| A7 | EQ Select Type | 00 01 02 | Auto PEQ GEQ EQ Off | AE | Zone3 Balance | 00 01 … 14 … 27 28 | L20 L19 Mid R19 R20 |
| A8 | Tone Bypass | 00 01 | Auto off |  |  |  |  |

28

RX-Vx800 RS-232C Protocol

| RCMD0, 1 | Report Item | RDAT0, 1 | Status | RCMD0, 1 | Report Item | RDAT0, 1 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B0 | Advanced Setup | 00 01 | Off On | B7 | Initialize | 00 01 02 03 04 | Cancel All DSP Parameter Video Network (only RX-V3800) |
| B1 | Remote ID for AMP | 00 01 | ID1 ID2 |  |  |  |  |
| B2 | Fan Control Mode | 00 01 | Auto Cont. |  |  |  |  |
|  |  |  |  | B8 | Video | 00 | Cancel |
| B3 | Speaker Impedance | 00 01 | 8 ohm 6 ohm |  | Reset | 01 | Yes |
|  |  |  |  | B9 | Remote Sensor | 00 01 | On Off |
| B4 | Tuner Step | 00 01 | AM10 / FM100 (kHz) AM 9 / FM50 (kHz) |  |  |  |  |
|  |  |  |  | BA | Remote ID for XM | 00 01 | ID1 ID2 |
| B5 | Remote ID for Tuner | 00 01 | ID1 ID2 |  |  |  |  |
|  |  |  |  | BB | Bi-AMP | 00 01 | On Off |
| B6 | Language | 00 01 02 03 04 05 | English Japanese French German Spanish Russian |  |  |  |  |
|  |  |  |  | BC | TV Format | 00 01 | PAL NTSC |
|  |  |  |  | BD | Wake on RS232C | 00 01 | No Yes |
|  |  |  |  | BE | Network | 00 | Cancel |
|  |  |  |  |  | Reset | 01 | Yes |
|  |  |  |  | BF | Monitor Check | 00 01 | Skip Yes |

Attention

* When the Input is changed, the RX-Vx800 sends an Operation Report for Input (RCMD0,1="21") and Audio Select

(RCMD0,1="22").

* When the System Memory is changed, the RX-Vx800 sends an Operation Report for System Memory

(RCMD0,1="30") and Configuration Command.

* When a headphone is plugged into the headphone jack and the Speaker Relay is turned off, the RX-Vx800 sends

an Operation Report for Speaker Relay A and B (RCMD0,1="2E","2F", RDAT="00(OFF)").

The RX-Vx800 sends an Operation Command for Speaker Relay A and B when the headphone is removed also.

* Each time the source from the Inputs or playback status (ex. 6.1/ES, RED DTS etc.) of the system changes, the

RX-Vx800 sends a Playback Status report.

* Each time the busy status of the system changes, the RX-Vx800 sends a System Status report.

29

RX-Vx800 RS-232C Protocol

5.4 Display Text Data Report

Command	Data

DDAT3

DC1	RCMD0 RCMD1  DDAT0  DDAT1  DDAT2	DDAT4  DDAT5  DDAT6  DDAT7	ETX

Data (Input name)

| RCMD0,1 | ITEM | DDAT0-1 | DDAT2-7 |
| --- | --- | --- | --- |
| 00 | Tuner Frequency | SP | 6digits Upper Lower |

| RCMD0,1 | ITEM | DDAT0 | DDAT1–7 |
| --- | --- | --- | --- |
| 01 | Main SP 7digits Upper Lower Volume |  |  |

| RCMD0,1 | ITEM | DDAT0 | DDAT1-7 |
| --- | --- | --- | --- |
| 02 | Zone2 Volume | SP | 7digits Upper Lower |

| RCMD0,1 | ITEM | DDAT0-8 |
| --- | --- | --- |
| 03 | Input name | 9letters <Right Left |

(example)

AM 1710kHz = 'SP' 'SP' '1' '7' '1' '0'

FM 108.5MHz = 'SP' 'SP' '1' '0' '8' '.' 5' '0'

(example)

-40.0dB = 'SP' '-' '4' '0' '.' '0' 'd' 'B'

| RCMD0, 1 | ITEM | DDAT0-8 |
| --- | --- | --- |
| 04 | Zone 2 Input name | 9letters <Right Left |

"DDAT8" is sent instead of "ETX". "ETX" is not sent.

"DDAT8" is sent instead of "ETX". "ETX" is not sent.

| RCMD0,1 | ITEM | DDAT0-2 | DDAT3-7 |
| --- | --- | --- | --- |
| 05 | Zone3 Volume | SP | 5digits Upper Lower |

| RCMD0, 1 | ITEM | DDAT0-8 |
| --- | --- | --- |
| 06 | Zone 3 Input name | 9letters <Right Left |

"DDAT8" is sent instead of "ETX". "ETX" is not sent.

| RCMD0,1 | ITEM | DDAT0-3 | DDAT4-7 |
| --- | --- | --- | --- |
| F0 | Remote Code | SP | 4digits Upper Lower |

(example)

'SP' 'SP' 'SP' 'SP' '7' 'A' '0' '0'

| RCMD0,1 | ITEM | DDAT0 | DDAT1-2 | DDAT3 | DDAT4 | DDAT5-6 | DDAT7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FF | Version Information | Major Version | Software Version | "_" (Don't care) | RS-232C Major Version | DSP Parameter Version | " " (Don't care) |

| FUNCTION | ITEM | DATA (ADCII) | RANGE |
| --- | --- | --- | --- |
| RCMD0,1 DDAT 0-7 | Command Data | 0-9, A-F 0-9, A-Z SP, other ASCII | 0...0xFF ASCII Space, dots |

30

RX-Vx800 RS-232C Protocol

Example of RX-Vx800 Control Procedure

[1]

Connection Start procedure (AC Plug / RS-232C cable connection)

When the AC plug / RS-232C cable are not connected, the 
RX-Vx800 cannot send any data to the host. If the host doesn't 
receive a configuration command after sending a Ready command 5 times, the host should disable the RS-232C communication and 
send an alert to the graphic user interface (GUI).

[1] : AC Plug / RS-232C connection check

(Start transaction)

Start

Send Ready command to RX-Vx800

[2]	AC plug / RS-232C connection check sequence after the connection has been confirmed in the procedure [1].

If the host doesn't receive a Report Command within 500ms of 
sending a command, the host should resend the command. If no Report Command is received after sending 5 times, check the 
AC plug/RS-232 cable ( cf. [1] ).

When the RS-232C cable is disconnected, the commands 
generated inside the RX-Vx800 are stored in the sending buffer.

If the stored commands exceed the buffer memory size (buffer 
overflow), the RX-Vx800 stops reporting any commands. In this 
case, reconnecting the AC plug or Connection Start procedure [1] will be needed in order to enable the command report.

Received Configuration

within 500msec ?

Yes

Connection is confirmed.

No

Already sent the Ready

Yes

Connection Failed !

Retry

No

[3]

[4]

AC plug connection detection ( after [1],[2] )

When the AC connection is reset, the RX-Vx800 sends a Configuration Command to the host. The host can display the status of the RX-Vx800 to its GUI.

Getting the status of the RX-Vx800 when the host boots up

At first, the host should send a Ready command and receive the Configuration Command from the RX-Vx800 ( cf. [1] ). Once the connection is confirmed, the host can send Control Commands to the RX-Vx800.

When the RX-Vx800 is turned off, it accepts only System Commands and Power ON command.

RX-Vx800

Configuration

Host

Ready Command

Report Command Enable ( Disable )

Control Command

[5]

Report Comman

Error transactions after [4]

d

While sending a control command, if the RX-Vx800 didn't send any corresponding Report Commands regardless of re-trying for 5 times, 
the host should clear its send buffer and then check the AC plug / RS-232C connection sequence (cf. [1]). When the RX-Vx800 responds, the host can display the RX-Vx800 status to its GUI then return to the normal communication sequence. If not, the host should cancel the communication and report the alert to its GUI.

31

RX-Vx800 RS-232C Protocol

Appendix

* ASCII Chart

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NUL | DLE | SP | 0 | @ | P | ` | p |
| 1 | SOH | DC1 | ! | 1 | A | Q | a | q |
| 2 | STX | DC2 | " | 2 | B | R | b | r |
| 3 | ETX | DC3 | # | 3 | C | S | c | s |
| 4 | EOT | DC4 | $ | 4 | D | T | d | t |
| 5 | ENQ | NAK | % | 5 | E | U | e | u |
| 6 | ACK | SYN | & | 6 | F | V | f | v |
| 7 | BEL | ETB | ' | 7 | G | W | g | w |
| 8 | BS | CAN | ( | 8 | H | X | h | x |
| 9 | HT | EM | ) | 9 | I | Y | I | y |
| A | LF | SUB | * | : | J | Z | j | z |
| B | VT | EXC | + | ; | K | [ | k | { |
| C | FF | FS | , | < | L | \ | l | ¦ |
| D | CR | GS | - | = | M | ] | m | } |
| E | SO | RS | . | > | N | ^ | n | ... |
| F | SI | US | / | ? | O | _ | o | DEL |

* Column numbers = the first hexadecimal digit 
  Row numbers = the second hexadecimal digit

* The characters in the gray cells are available for RS-232C communication.

32